# QIDI Plus 4 Max - Moonraker Connection Guide

## Complete Setup & Troubleshooting for macOS (Nick’s Network: 192.168.50.x)

## Table of Contents

- Quick Start Checklist

- Finding the Printer’s IP Address

- WiFi Network Setup

- Checking if Moonraker is Running

- Opening Port 7125

- SSH Access to the Printer

- USB Connection (Fallback)

- mDNS Auto-Discovery Setup

- Running the Python Client

- Setting Up the MCP Server

- Calibration Guide

- Common Issues & Solutions

- Moonraker API Reference

- Useful G-Codes for QIDI Plus 4 Max

## 1. Quick Start Checklist

Before diving into detailed troubleshooting, run through this checklist:

- ☐ Printer is powered on and booted (touchscreen shows main menu)

- ☐ Printer is connected to your WiFi network (same as your Mac)

- ☐ You know the printer’s IP address

- ☐ Your Mac can ping the printer’s IP

- ☐ Moonraker service is running on the printer

- ☐ Port 7125 is not blocked by a firewall

If all checks pass, run:

python moonraker_client.py --ip YOUR_PRINTER_IP status

## 2. Finding the Printer’s IP Address

### Method 1: From the Touchscreen (Easiest)

- On the QIDI Plus 4 Max touchscreen, tap Settings (gear icon)

- Go to Network or WiFi Settings

- Look for IP Address - it will show something like 192.168.50.xxx

- Write this down - this is your MOONRAKER_IP

QIDI printers often show the IP on the main screen or in a network info widget. Look for a small WiFi icon with numbers.

### Method 2: From Your Router Admin Panel

- Open a browser on your Mac

- Navigate to your router’s admin page (usually http://192.168.50.1 or http://192.168.1.1)

- Log in (check router sticker for credentials)

- Go to Connected Devices or DHCP Client List

- Look for a device named:

  - QIDI

  - qidi-plus4

  - klipper

  - Or a MAC address starting with the printer’s vendor prefix

- Note the IP address

### Method 3: Using arp-scan on macOS

Install and run arp-scan to find devices on your network:

# Install arp-scan (if not already installed)
brew install arp-scan

# Scan the network (replace en0 with your active interface)
sudo arp-scan --localnet --interface=en0

# Or scan specific range
sudo arp-scan 192.168.50.1-192.168.50.254

Look for entries with vendor names like Qidi, Raspberry Pi, or similar.

### Method 4: Using Python Client’s Discovery

# Auto-scan the network
python moonraker_client.py discover --subnet 192.168.50

This scans all IPs from .1 to .254 on port 7125 and reports any Moonraker instances found.

To scan multiple subnets:

python moonraker_client.py discover --subnet 192.168.1
python moonraker_client.py discover --subnet 10.0.0

## 3. WiFi Network Setup

### If the Printer is NOT on WiFi Yet

The QIDI Plus 4 Max needs to be connected to your 2.4GHz WiFi network (most 3D printers don’t support 5GHz).

#### Option A: Via Touchscreen

- On the printer touchscreen, go to Settings > WiFi

- Select your network name (SSID)

- Enter your WiFi password

- Wait for it to connect - you should see an IP address appear

#### Option B: Via USB Connection (if touchscreen method fails)

- Connect your Mac to the printer via USB-C cable

- The printer should appear as a serial device

- Use a terminal program (like screen or minicom) to connect:

# Find the serial device
ls /dev/tty.*
# Look for something like /dev/tty.usbserial-XXXX or /dev/ttyACM0

# Connect (replace with your device)
screen /dev/tty.usbserial-XXXX 115200

- Once connected, you can use the Klipper console to configure WiFi:

# For printers with network manager
WIFI_CONNECT SSID="YourNetworkName" PASSWORD="YourPassword"

#### Option C: Edit Configuration Directly (Advanced)

If you have SSH access (see Section 6):

ssh root@PRINTER_IP
# Edit the WiFi config (location varies, common paths):
nano /etc/wpa_supplicant.conf
# or
nano /etc/network/interfaces
# Restart networking
systemctl restart networking

### Ensuring Same Network/Subnet

Your Mac and the printer MUST be on the same network subnet to communicate.

Check your Mac’s IP:

ipconfig getifaddr en0
# Output example: 192.168.50.100

The first three octets must match the printer’s IP (e.g., both start with 192.168.50).

If they’re on different subnets, either: 1. Move the printer to the same WiFi network as your Mac 2. Or configure your router to allow inter-subnet communication

## 4. Checking if Moonraker is Running

Once you have the printer’s IP, test if Moonraker is accessible:

### From Your Mac (Terminal)

# Test basic connectivity
ping 192.168.50.xxx

# Test if port 7125 is open
nc -zv 192.168.50.xxx 7125
# or
telnet 192.168.50.xxx 7125

### Test Moonraker API Directly

# Get server info
curl http://192.168.50.xxx:7125/server/info

# Get printer status
curl 'http://192.168.50.xxx:7125/printer/objects/query?objects=toolhead&objects=extruder'

# List files
curl http://192.168.50.xxx:7125/server/files/list?root=gcodes

Expected response (server info):

{
  "result": {
    "version": "v0.8.0",
    "klippy_connected": true,
    "components": ["database", "file_manager", "klippy_apis", "machine"]
  }
}

### If Moonraker is NOT Responding

#### Check from the Printer (via SSH)

ssh root@192.168.50.xxx

# Check if Moonraker service is running
systemctl status moonraker

# If not running, start it
systemctl start moonraker

# Enable auto-start
systemctl enable moonraker

# Check Moonraker logs
journalctl -u moonraker -f
# or
tail -f /tmp/moonraker.log

#### Restart Moonraker Manually

ssh root@192.168.50.xxx

# Restart the service
systemctl restart moonraker

# Or kill and restart manually
pkill -f moonraker
moonraker -c /etc/moonraker.conf

## 5. Opening Port 7125

### Check if the Port is Open

From your Mac:

# Check if port 7125 is reachable
nmap -p 7125 192.168.50.xxx

# Or use netcat
nc -zv 192.168.50.xxx 7125

### If the Port is Blocked

#### A. Check Printer Firewall (via SSH)

ssh root@192.168.50.xxx

# Check if firewall is active
iptables -L -n | grep 7125

# If blocked, add a rule to allow port 7125
iptables -A INPUT -p tcp --dport 7125 -j ACCEPT

# Save rules (method varies by Linux distro)
iptables-save > /etc/iptables/rules.v4

#### B. Check moonraker.conf

ssh root@192.168.50.xxx
cat /etc/moonraker.conf

Look for the [server] section:

[server]
host: 0.0.0.0
port: 7125
# Make sure host is 0.0.0.0 (accept all) not 127.0.0.1 (localhost only)

If host is set to 127.0.0.1, change it to 0.0.0.0:

nano /etc/moonraker.conf
# Change: host: 0.0.0.0
systemctl restart moonraker

#### C. Check Router Firewall

Some routers have client isolation that blocks devices from talking to each other.

- Log into your router admin (http://192.168.50.1)

- Look for settings like:

  - “AP Isolation” - DISABLE this

  - “Client Isolation” - DISABLE this

  - “Guest Network Isolation” - Don’t connect printer to guest network

#### D. macOS Firewall

Your Mac’s firewall usually doesn’t block outbound connections, but check:

# Check firewall status
sudo /usr/libexec/ApplicationFirewall/socketfilterfw --getglobalstate

# If needed, temporarily disable
sudo /usr/libexec/ApplicationFirewall/socketfilterfw --setglobalstate off
# Re-enable after testing
sudo /usr/libexec/ApplicationFirewall/socketfilterfw --setglobalstate on

## 6. SSH Access to the Printer

Most QIDI Plus 4 Max printers run a Linux-based SBC (Single Board Computer) like a Raspberry Pi or similar. SSH gives you full control.

### Default SSH Credentials (QIDI)

| Field | Value |
| --- | --- |
| Username | root or qidi |
| Password | root or qidi or klipper |
| Port | 22 |

### Connecting via SSH

# Try these combinations
ssh root@192.168.50.xxx
# Password: root

ssh qidi@192.168.50.xxx
# Password: qidi

# If those don't work, try common defaults
ssh root@192.168.50.xxx
# Password: klipper
# Password: 123456
# Password: (empty - just press Enter)

### Finding the Correct Credentials

If defaults don’t work: 1. Check QIDI documentation/manual 2. Check QIDI’s official website/support 3. Try the touchscreen - some printers show SSH info in Settings > About 4. Check online forums for QIDI Plus 4 Max SSH credentials

### Useful SSH Commands Once Connected

# Check system info
uname -a
cat /etc/os-release

# Check running services
systemctl list-units --type=service

# Check Moonraker specifically
systemctl status moonraker
journalctl -u moonraker --no-pager -n 50

# Check Klipper
systemctl status klipper

# View printer configuration
cat ~/printer_data/config/printer.cfg

# Restart services
systemctl restart moonraker
systemctl restart klipper

# Check disk space
df -h

# Check network
ip addr show
ifconfig

## 7. USB Connection (Fallback)

If WiFi/network connection fails entirely, use USB as a fallback.

### macOS USB Connection

- Connect the QIDI Plus 4 Max to your Mac via USB cable

- Open Terminal and find the device:

# List serial devices
ls /dev/tty.*
ls /dev/cu.*

# Common names:
# /dev/tty.usbserial-XXXX
# /dev/tty.usbmodemXXXX
# /dev/cu.usbserial-XXXX
# /dev/cu.wchusbserialXXXX

- Use screen to connect:

screen /dev/tty.usbserial-XXXX 115200

- Press Enter - you should see a Klipper console prompt

### Using the Client with USB (Serial Bridge)

Some setups expose Moonraker over a USB serial bridge. If so, the API might be available at a different endpoint. Check if your QIDI printer supports this.

### Alternative: USB-to-Ethernet

If the printer has an Ethernet port (check the back): 1. Connect an Ethernet cable directly from printer to Mac (or via router) 2. The printer may get an IP via DHCP 3. Check the IP on the touchscreen or router

## 8. mDNS Auto-Discovery Setup

mDNS (Bonjour/Avahi) lets your printer auto-discover on the network without knowing its IP.

### Enable mDNS on the Printer (via SSH)

ssh root@192.168.50.xxx

# Install avahi (if not present)
apt-get update && apt-get install -y avahi-daemon

# Create mDNS service file for Moonraker
cat > /etc/avahi/services/moonraker.service << 'EOF'
<?xml version="1.0" standalone='no'?>
<!DOCTYPE service-group SYSTEM "avahi-service.dtd">
<service-group>
  <name replace-wildcards="yes">%h Moonraker</name>
  <service>
    <type>_http._tcp</type>
    <port>7125</port>
    <txt-record>path=/</txt-record>
  </service>
</service-group>
EOF

# Restart avahi
systemctl restart avahi-daemon
systemctl enable avahi-daemon

### Discover from macOS

# Browse for Moonraker services
dns-sd -B _http._tcp

# Or use the Python client with mDNS
python -c "
import socket
try:
    result = socket.getaddrinfo('qidi-plus4.local', 7125)
    print('Found:', result[0][4][0])
except:
    print('mDNS name not found')
"

### Using .local Hostname

After setting up mDNS, you can connect using:

python moonraker_client.py --ip qidi-plus4.local status

## 9. Running the Python Client

### Installation

# The client uses only Python standard library - no pip install needed!
# Just download and run
chmod +x moonraker_client.py

### Basic Commands

# 1. Discover printer on network
python moonraker_client.py discover --subnet 192.168.50

# 2. Check status
python moonraker_client.py --ip 192.168.50.xxx status

# 3. Check temperatures
python moonraker_client.py --ip 192.168.50.xxx temps

# 4. List files
python moonraker_client.py --ip 192.168.50.xxx list

# 5. Upload a file
python moonraker_client.py --ip 192.168.50.xxx upload ~/Downloads/benchy.gcode

# 6. Start a print
python moonraker_client.py --ip 192.168.50.xxx print benchy.gcode

# 7. Monitor print progress
python moonraker_client.py --ip 192.168.50.xxx monitor

# 8. Pause/Resume/Cancel
python moonraker_client.py --ip 192.168.50.xxx pause
python moonraker_client.py --ip 192.168.50.xxx resume
python moonraker_client.py --ip 192.168.50.xxx cancel

### Save IP for Convenience

After discovery, the client saves the IP automatically. Then you can omit --ip:

python moonraker_client.py status
python moonraker_client.py temps

## 10. Setting Up the MCP Server

### Prerequisites

# Install the MCP Python SDK
pip install mcp

### Configure for Claude Code

Edit your Claude Code settings file:

# Location on macOS
mkdir -p ~/Library/Application\ Support/Claude
nano ~/Library/Application\ Support/Claude/settings.json

Add the Moonraker MCP server:

{
  "mcpServers": {
    "moonraker": {
      "command": "python3",
      "args": ["/path/to/moonraker_mcp_server.py"],
      "env": {
        "MOONRAKER_IP": "192.168.50.xxx",
        "MOONRAKER_PORT": "7125"
      }
    }
  }
}

Replace /path/to/moonraker_mcp_server.py with the actual path and 192.168.50.xxx with your printer’s IP.

### Start the MCP Server Manually (for testing)

export MOONRAKER_IP=192.168.50.xxx
python3 moonraker_mcp_server.py

### Using with Claude Code

Once configured, you can ask Claude things like:

- “What’s the current temperature of my printer?”

- “Start printing the calibration cube file”

- “Pause the current print”

- “Home all axes”

- “Run PID tune on the extruder at 210 degrees”

- “List all G-code files on the printer”

- “Upload this file and start printing it”

### MCP Tools Available

| Tool | Description |
| --- | --- |
| printer_status | Full printer status (position, temps, state, extruder) |
| query_temperatures | Just temperature readings |
| start_print | Start a print job (specify filename) |
| pause_print | Pause current print |
| resume_print | Resume paused print |
| cancel_print | Cancel/abort print |
| home_axes | Home X/Y/Z axes |
| run_gcode | Execute any G-code command |
| upload_file | Upload G-code from your Mac |
| list_files | List files on printer |
| emergency_stop | Emergency stop (M112) |
| firmware_restart | Restart Klipper firmware |
| pid_tune | PID autotune for a heater |
| calibrate_bed_mesh | Bed mesh calibration |
| set_temperature | Set heater target temperature |
| get_active_extruder | Get current active extruder |
| set_active_extruder | Switch extruder (dual head) |

## 11. Calibration Guide

### Full First-Time Setup

# Run the automated setup (this takes ~30-40 minutes)
python moonraker_client.py --ip 192.168.50.xxx setup

This runs: 1. Connection test 2. XYZ homing 3. PID tune extruder 0 (200C) 4. PID tune extruder 1 (200C) - if dual extruder 5. PID tune bed (60C) 6. Bed mesh calibration 7. Pressure advance calibration

### Individual Calibration Steps

#### PID Tune (Hotend)

# Tune extruder at 200C (PLA)
python moonraker_client.py --ip 192.168.50.xxx pid --heater extruder --target 200

# Tune extruder at 250C (ABS/ASA)
python moonraker_client.py --ip 192.168.50.xxx pid --heater extruder --target 250

# Tune bed at 60C
python moonraker_client.py --ip 192.168.50.xxx pid --heater heater_bed --target 60

Important: After PID tune completes, you MUST save config:

python moonraker_client.py --ip 192.168.50.xxx gcode --script "SAVE_CONFIG"

#### Bed Mesh Calibration

# Heat bed first (to print temperature)
python moonraker_client.py --ip 192.168.50.xxx gcode --script "M140 S60"

# Wait for bed to reach temp (check with temps command)
python moonraker_client.py --ip 192.168.50.xxx temps

# Run bed mesh
python moonraker_client.py --ip 192.168.50.xxx bedmesh --profile default

# Save the mesh
python moonraker_client.py --ip 192.168.50.xxx gcode --script "BED_MESH_PROFILE SAVE=default"
python moonraker_client.py --ip 192.168.50.xxx gcode --script "SAVE_CONFIG"

#### Pressure Advance

# Run pressure advance calibration
python moonraker_client.py --ip 192.168.50.xxx pressure-advance --extruder extruder

If the macro isn’t found, print a PA calibration tower and manually find the best PA value, then set it in your slicer or printer.cfg.

## 12. Common Issues & Solutions

### Issue: “Connection refused” or “No route to host”

Causes & Fixes: 1. Wrong IP - Double-check the IP on the touchscreen 2. Different subnet - Mac and printer must share first 3 octets 3. Printer not on WiFi - Connect via touchscreen first 4. Firewall blocking - Check router and Mac firewall settings 5. Moonraker not running - SSH in and systemctl start moonraker

### Issue: “Moonraker found but Klipper not connected”

# Check Klipper status
ssh root@192.168.50.xxx systemctl status klipper

# Restart Klipper
ssh root@192.168.50.xxx systemctl restart klipper

# Check for config errors
ssh root@192.168.50.xxx journalctl -u klipper -n 20

### Issue: Upload fails / times out

- Check disk space: ssh root@192.168.50.xxx df -h

- Check file permissions

- Try smaller file first

- Increase timeout in the client code

### Issue: Print won’t start

- Check if printer is homed: python moonraker_client.py status

- Home axes first: python moonraker_client.py home

- Check if file exists: python moonraker_client.py list

- Check bed mesh is loaded

### Issue: Temperatures read 0C

- Check thermistor connections inside printer

- Check printer.cfg has correct thermistor pins

- Restart Klipper after config changes

### Issue: Dual extruder not working

# Check which extruder is active
python moonraker_client.py --ip 192.168.50.xxx toolhead

# Switch to extruder 1
python moonraker_client.py --ip 192.168.50.xxx extruder --set extruder1

# Check extruder config in printer.cfg via SSH
ssh root@192.168.50.xxx cat ~/printer_data/config/printer.cfg | grep -A5 "extruder1"

## 13. Moonraker API Reference

### Core Endpoints

| Endpoint | Method | Description |
| --- | --- | --- |
| /server/info | GET | Server version, components, Klipper connection state |
| /printer/objects/query | GET | Query any printer object (toolhead, temps, etc.) |
| /printer/print/start | POST | Start printing a file |
| /printer/print/pause | POST | Pause current print |
| /printer/print/resume | POST | Resume paused print |
| /printer/print/cancel | POST | Cancel current print |
| /printer/gcode/script | POST | Execute G-code |
| /printer/firmware_restart | POST | Restart Klipper firmware |
| /server/files/upload | POST | Upload a file |
| /server/files/list | GET | List files in a directory |
| /server/files/delete | DELETE | Delete a file |
| /machine/reboot | POST | Reboot the host system |

### Query Objects

Common objects to query via /printer/objects/query?objects=NAME:

| Object | Data Provided |
| --- | --- |
| toolhead | Position, active extruder, homed axes |
| extruder | Temperature, pressure advance, target |
| extruder1 | Second extruder data |
| heater_bed | Bed temperature, target |
| print_stats | Print state, filename, progress, duration |
| virtual_sdcard | SD card/virtual file progress |
| gcode_move | Current position, speed, extrude factor |
| fan | Fan speed |
| display_status | Display/progress info |

## 14. Useful G-Codes for QIDI Plus 4 Max

### Temperature Control

M104 S200          ; Set hotend temp (no wait)
M109 S200          ; Set hotend temp and wait
M140 S60           ; Set bed temp (no wait)
M190 S60           ; Set bed temp and wait
M140 S0 / M104 S0  ; Turn off heaters

### Movement

G28                ; Home all axes
G28 X              ; Home X only
G1 X10 Y10 F3000   ; Move to position at speed
G1 Z5              ; Move Z axis
G91                ; Relative positioning
G90                ; Absolute positioning

### Extrusion

M83                ; Relative extrusion mode
G1 E10 F300        ; Extrude 10mm
G1 E-5 F300        ; Retract 5mm
G1 E-40 F1800      ; Full filament retract (for change)

### Calibration

PID_CALIBRATE HEATER=extruder TARGET=200   ; PID tune
BED_MESH_CALIBRATE                         ; Bed mesh
BED_MESH_PROFILE SAVE=default              ; Save mesh
SAVE_CONFIG                                ; Save all settings

### Information

M115               ; Firmware info
M503               ; Current settings
M105               ; Temperature report

## Need More Help?

- Klipper docs: https://www.klipper3d.org/

- Moonraker docs: https://moonraker.readthedocs.io/

- QIDI support: Check QIDI’s official support channels

- Reddit: r/klippers, r/3Dprinting

- Discord: Klipper Discord server