# MEOK Labs - Complete Parts List for 3D-Printed Robot Army

## Comprehensive Research: Actuators, Grippers, Sensors & Electronics

Compiled for: Nick @ MEOK Labs | Printer: QIDI Plus 4 Max
Date: June 2026 | Sources: AliExpress, Amazon, RobotShop, ServoCity, Adafruit, SparkFun, goBILDA

# PART 1: ACTUATORS & SERVOS

## 1.1 DYNAMIXEL SERIES (Gold Standard - Expensive)

The ROBOTIS DYNAMIXEL family is the gold standard for robotic actuators. All models feature position/velocity/torque feedback, daisy-chain networking (TTL/RS485), and PID control. They work seamlessly with ROS2 via dynamixel_sdk.

### XL Series (Entry-Level, Plastic Gears)

| Model | Price | Source | Stall Torque | Speed | Voltage | Weight | Key Specs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| XL330-M288-T | $23.90 | RobotShop, Dynamixel.com | 0.93 N.m | 81 rpm | 5.0V | 23g | Ultra-compact, 6 control modes, current control |
| XL330-M077-T | $23.90 | RobotShop, Dynamixel.com | 0.22 N.m | 363 rpm | 5.0V | 23g | High-speed version, same size |
| XL430-W250-T | $23.90-$39 | RobotShop, robosavvy.co.uk | 1.5 N.m | 61 rpm | 6.5-12V | 57g | Best entry-level value, 360 deg control, ABS encoder |
| 2XL430-W250-T | $129.90 | Dynamixel.com | 1.4 N.m | 57 rpm | 11.1V | 98g | Dual-axis actuator for compact joints |

### XM/XC Series (Mid-Range, Metal Gears)

| Model | Price | Source | Stall Torque | Speed | Voltage | Weight | Key Specs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| XC430-W150-T | $119.90 | Dynamixel.com | 1.6 N.m | 106 rpm | 12V | 65g | Speed-optimized, metal gears |
| XC430-W240-T | $119.90 | Dynamixel.com | 1.9 N.m | 70 rpm | 12V | 65g | Torque-optimized variant |
| XM430-W210-T | $269.90 | Dynamixel.com | 3.0 N.m | 77 rpm | 12V | 82g | Best for medium joints |
| XM430-W350-T | $269.90 | Dynamixel.com | 4.1 N.m | 46 rpm | 12V | 82g | High-torque variant |
| XM540-W150-T | $419.90 | Dynamixel.com | 7.3 N.m | 53 rpm | 12V | 165g | Heavy-duty joints |
| XM540-W270-T | $419.90 | Dynamixel.com | 10.6 N.m | 30 rpm | 12V | 165g | Maximum XM torque |

### XH/XD Series (High-End)

| Model | Price | Source | Stall Torque | Speed | Voltage | Weight | Key Specs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| XH430-W350-T | $359.90 | Dynamixel.com | 3.4 N.m | 30 rpm | 12V | 82g | High precision, low backlash |
| XD540-T150-R | $769.90 | Dynamixel.com | 7.1 N.m | 70 rpm | 12V | 170g | Industrial grade |
| XD540-T270-R | $769.90 | Dynamixel.com | 9.9 N.m | 39 rpm | 12V | 170g | Maximum XD torque |
| XH540-W150-R | $549.90 | Dynamixel.com | 7.1 N.m | 70 rpm | 12V | 165g | Widely used in research |

### Integration Notes

- Protocol: XL/XM use Protocol 2.0 (TTL half-duplex serial) - daisy-chain capable

- Controller: U2D2 ($49.90) or OpenCM9.04 ($9.90) for USB-to-Dynamixel bridge

- ROS2: dynamixel_sdk + dynamixel_workbench packages available

- Power: 12V SMPS recommended (3S LiPo compatible)

- Where to buy cheapest: RobotShop (EU), robosavvy (UK), or bulk from Robotis directly

## 1.2 CHEAP ALIEXPRESS SERVOS (Budget Workhorses)

For the MEOK army on a budget, these servos offer incredible value. All are 3D-printable-compatible and widely available.

| Model | Price (AliExpress) | Stall Torque | Speed | Voltage | Weight | Gear Material | Waterproof |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TowerPro SG90 | $1.50-$3.00 | 1.8 kg.cm (0.18 N.m) | 0.12s/60deg | 4.8-6V | 9g | POM plastic | No |
| TowerPro MG90S | $3.00-$5.00 | 2.2 kg.cm (0.22 N.m) | 0.08s/60deg | 4.8-6V | 14g | Metal | No |
| MG996R | $3.50-$6.00 | 11 kg.cm (1.1 N.m) @6V | 0.14s/60deg | 4.8-7.2V | 55g | Metal | No |
| MG995 | $3.00-$5.00 | 10 kg.cm (1.0 N.m) @6V | 0.20s/60deg | 4.8-7.2V | 55g | Metal | No |
| DS3218 (20kg) | $8.00-$15.00 | 20 kg.cm (2.0 N.m) @6.8V | 0.14s/60deg | 4.8-6.8V | 60g | Copper+Aluminum | IP66 |
| DS3225 (25kg) | $10.00-$17.00 | 25 kg.cm (2.45 N.m) @6V | 0.13s/60deg | 4.8-7.4V | 62g | Metal | IP66 |
| DS3235 (35kg) | $12.00-$20.00 | 35 kg.cm (3.43 N.m) @6.8V | 0.12s/60deg | 4.8-6.8V | 65g | Metal | IP66 |
| DS3240 (40kg) | $15.00-$25.00 | 40 kg.cm (3.92 N.m) @6.8V | 0.10s/60deg | 4.8-6.8V | 68g | Full metal | IP66 |
| JX BLS-HV7132MG | $25.00-$35.00 | 32 kg.cm (3.14 N.m) | 0.07s/60deg | 6.0-8.4V | 70g | Steel, Brushless | Yes |

### Torque Comparison (Humanoid Context)

| Joint Type | Recommended Torque (N.m) | Budget Servo Choice | Premium Choice |
| --- | --- | --- | --- |
| Gripper/finger | 0.2 - 0.5 | SG90 / MG90S | XL330-M288 |
| Wrist | 0.5 - 1.5 | MG996R | XL430-W250 |
| Elbow | 2.0 - 4.0 | DS3218 / DS3225 | XM430-W350 |
| Shoulder | 4.0 - 8.0 | DS3235 / DS3240 | XM540-W150 |
| Hip (humanoid) | 6.0 - 10.0 | 2x DS3240 (paired) | XM540-W270 |
| Knee | 4.0 - 8.0 | DS3235 / DS3240 | XM540-W150 |
| Ankle | 2.0 - 4.0 | DS3225 / DS3235 | XM430-W350 |
| Quadruped leg | 3.0 - 6.0 | DS3235 | XM430-W350 |

### Integration Notes

- PWM frequency: Standard 50Hz (20ms period), pulse 500-2500us for 180deg

- Driver: PCA9685 16-channel I2C driver ($2-5 on AliExpress)

- Power budget: Plan ~1A per servo at stall (MG996R/DS3218), use external 5-6V BEC

- 3D-printed horns: Most use 25T spline - design custom horns in CAD

- Heat: Metal gear servos run hot under continuous load - add ventilation

## 1.3 STEPPER MOTORS

| Model | Price (AliExpress) | Holding Torque | Current/Phase | Weight | Key Specs |
| --- | --- | --- | --- | --- | --- |
| NEMA 17 (42mm) | $8.00-$15.00 | 0.4-0.9 N.m (40-90 N.cm) | 1.5-2.0A | 270-350g | Standard for 3D printers |
| NEMA 17 Long (48mm) | $12.00-$20.00 | 0.52 N.m (52 N.cm) | 1.5A | 350g | Better for robot arms |
| NEMA 17 Extended (60mm) | $15.00-$25.00 | 0.65 N.m (65 N.cm) | 2.0A | 400g | High torque variant |
| NEMA 23 (57mm) | $18.00-$35.00 | 1.2-2.5 N.m | 2.8-4.0A | 600-1000g | Heavy-duty joints |
| NEMA 23 Long (76mm) | $25.00-$45.00 | 1.8-3.0 N.m | 3.0-4.0A | 900-1200g | CNC-grade torque |

### Stepper Drivers

| Driver | Price | Max Current | Microstepping | Features |
| --- | --- | --- | --- | --- |
| A4988 | $1.00-$2.50 | 1A/phase | 1/16 | Basic, cheap |
| DRV8825 | $2.00-$4.00 | 2.2A/phase | 1/32 | Better current control |
| TMC2209 | $4.00-$8.00 | 2.8A/phase | 1/256 | Silent, UART config, stallGuard |
| TMC5160 | $12.00-$20.00 | 4.4A/phase | 1/256 | SPI, highest performance |

### Integration Notes

- Robot arm use: Steppers are great for elbows/shoulders where you need precise position but not fast response

- Closed-loop: Add AS5600 magnetic encoder ($2-5) for closed-loop stepper control

- Gear reduction: 5:1 to 10:1 planetary gearbox ($15-30) dramatically increases effective torque

- ROS2: Use ros2_control with stepper motor controllers

## 1.4 BLDC MOTORS + ODrive

For advanced, high-torque, backdrivable joints, BLDC motors with ODrive controllers are the ultimate choice.

### BLDC Motors

| Model | Price | KV Rating | Max Torque | Voltage | Weight | Source |
| --- | --- | --- | --- | --- | --- | --- |
| Turnigy Aerodrive 5065 | $30-$50 | 270KV | ~2.5 N.m | 24V | 350g | HobbyKing |
| D5312S-330KV | ~$15/motor | 330KV | ~1.5 N.m | 24V | 280g | AliExpress |
| ODrive D6374 | $89 | 150KV | ~5.0 N.m | 48V | 850g | ODrive Robotics |
| T-Motor U8 Lite | $120-$150 | 100KV | ~8.0 N.m | 48V | 400g | T-Motor |
| Hoverboard motor | $15-$30/pair | 15-20KV | ~10 N.m | 36V | 2.5kg each | AliExpress |

### ODrive Controllers

| Model | Price | Max Power | Voltage | Current/Motor | Channels |
| --- | --- | --- | --- | --- | --- |
| ODrive S1 | $199-$249 | 1600W | 12-50V | 40A cont | 1 motor |
| ODrive Pro | $349-$449 | 3000W | 14-58V | 80A cont | 1 motor |
| ODrive Micro | ~$79 | 300W | 12-40V | 20A cont | 1 motor |
| ODESC 3.6 (ODrive clone) | $50-$80 | 1000W | 12-50V | 30A cont | 1 motor |
| Makerbase ODrive | ~$30-$50 | 500W | 12-40V | 20A cont | 1 motor |
| ODrive v3.6 (legacy) | ~$99-$149 | 1000W | 12-56V | 40A cont | 2 motors |

### Integration Notes

- Encoder required: AS5600 ($2), AMT102 ($30), or TLE5012 ($15) for position feedback

- Backdrivable: Excellent for compliant control and human interaction

- ROS2: odriveros2 package available, ros2_control integration

- Power: Needs substantial power supply - 24V/10A minimum for most setups

- For humanoid: Use 2x ODrive Pro + 4x D6374 for hips/knees = ~$1500 just for actuation

- Budget option: Hoverboard motors ($15/pair) + Makerbase ODrive ($40) = incredible torque per dollar

## 1.5 BEST SERVO FOR EACH APPLICATION (Quick Reference)

| Application | Budget Choice | Mid-Tier | Premium | Notes |
| --- | --- | --- | --- | --- |
| Humanoid finger | SG90 ($2) | MG90S ($4) | XL330-M288 ($24) | SG90 is fine for simple grippers |
| Humanoid wrist | MG996R ($5) | DS3218 ($12) | XL430-W250 ($24) | XL430 has 360deg control |
| Humanoid elbow | DS3225 ($14) | DS3235 ($17) | XM430-W210 ($270) | 2.5-4 N.m needed |
| Humanoid shoulder | DS3235 ($17) | DS3240 ($20) | XM430-W350 ($270) | 3.5-6 N.m needed |
| Humanoid hip | 2x DS3240 ($40) | DS3240+gearbox | XM540-W270 ($420) | 6-10 N.m needed |
| Humanoid knee | DS3235 ($17) | DS3240 ($20) | XM540-W150 ($420) | 5-8 N.m needed |
| Quadruped leg (DOF) | DS3235 ($17) | DS3240 ($20) | XM430-W350 ($270) | 3-4 N.m is enough per joint |
| Quadruped (BLDC) | Hoverboard motor ($8) | D5312S+ODrive S1 ($215) | D6374+ODrive Pro ($540) | BLDC is the gold standard |
| Gripper finger | SG90 ($2) | MG90S ($4) | Custom BLDC ($50) | Simple grippers need little torque |
| Pan/tilt camera | SG90x2 ($4) | MG90Sx2 ($8) | XL330x2 ($48) | Speed matters for tracking |

# PART 2: GRIPPERS

## 2.1 Servo-Driven Grippers (3D Printable)

| Design | Actuator | Price to Build | Grip Force | Best For | STL/Source |
| --- | --- | --- | --- | --- | --- |
| Simple Parallel Jaw | 1x MG996R | $5-8 | ~5N | Pick & place, blocks | Thingiverse: “Servo Gripper” |
| 3-Finger Adaptive (MDPI) | 3x micro servo | $15-25 | ~8N | Irregular objects | Paper: “Open-Source 3D Printed Three-Fingered Robotic Gripper” |
| 9g Servo Gripper | 1x SG90 | $3-5 | ~2N | Lightweight, teaching | Thingiverse: “SG90 Gripper” |
| Robotis RH-P12-RN | Dynamixel | $499 | 15N | Research, force control | RobotShop |
| Robotis Hand Gripper | 2x XL430 | $120 (parts) | 10N | Precise manipulation | Custom STL on GitHub |

### Recommended 3D-Printable Gripper Designs

- “Parametric Robot Gripper” (Thingiverse #3847625) - Single servo, parallel jaw, fully parametric OpenSCAD

- “3-Finger Adaptive Gripper” (MDPI Biomimetics 2025) - 3D printed in PLA+TPU, 3 micro servos, $15 total

- “MeArm” - Classic 4-DOF mini arm + gripper, SG90 servos, $20 total, full open source

- “EEZYbotARM MK2” - 4-DOF arm with parallel gripper, MG996R servos, $30 total

- “JPL Open Source Rover Gripper” - Mars rover style, 1 servo, very robust

## 2.2 Adaptive/Flexure Grippers (No Motors, Compliant)

| Design | Material | Price | Principle | Best For |
| --- | --- | --- | --- | --- |
| Compliant Finger | PLA/TPU | $2-5 | Elastic deformation | Lightweight, simple pick |
| Fin Ray Effect | TPU 95A | $3-8 | Bio-inspired ribs | Conforming to object shape |
| Origami Gripper | Paper+3D printed base | $1-3 | Foldable mechanism | Ultra-low cost |
| Vacuum-Assisted | TPU + aquarium pump | $10-15 | Suction + compliance | Flat objects, pick & place |
| Gecko-inspired | PLA + adhesive pads | $5-10 | Dry adhesion | Smooth surfaces |

### Key Insight

Flexure grippers printed in TPU 95A can grip a wide variety of object shapes with zero motors - just use a single linear actuator or servo to open/close the compliant mechanism. The “fin ray effect” (bio-inspired rib structure) causes fingers to conform to object shape automatically.

## 2.3 Soft Robotics Grippers (Pneumatic)

| Component | Price | Source | Notes |
| --- | --- | --- | --- |
| 3D Printed Mold | $5 (filament) | Print yourself | Design in Fusion 360, print mold in PLA |
| Food-safe Silicone | $15-25/kg | Smooth-On Ecoflex 00-30 | Shore 00-30 is ideal soft gripper |
| Mini Air Pump | $5-10 | AliExpress 12V/370 pump | 2-3 PSI is enough |
| Solenoid Valve | $3-5 each | AliExpress 2-way valve | 5V or 12V |
| Pressure Sensor | $5-10 | MPX5700 or similar | For closed-loop control |
| Complete Soft Gripper | $20-40 total | DIY | Delta X Soft Gripper design |

### Design Tips for 3D-Printed Soft Grippers

- Mold design: Print mold halves in PLA, coat with mold release spray

- Wall thickness: 1.5-2mm silicone walls give best compliance

- Chambers: 3-5 pneumatic chambers per finger

- Operating pressure: 10-40 kPa (very low - safe for humans)

- Best source: Delta X Soft Gripper (open source, ~$10 to build)

## 2.4 Vacuum Grippers

| Component | Price | Source | Notes |
| --- | --- | --- | --- |
| Mini Vacuum Pump | $5-12 | AliExpress (12V 370 pump) | Suction cup attachment |
| Suction Cups (set) | $3-8 | AliExpress (various sizes) | 10-40mm diameter |
| Solenoid Valve | $3-5 | AliExpress 2-way | For vacuum release |
| Vacuum Sensor | $5-10 | MPXV7002DP or similar | Detect if object gripped |
| DIY Vacuum Ejector | $2-5 | 3D print + Venturi | Uses compressed air |
| Complete Vacuum Gripper | $15-30 total | DIY | Best for flat objects |

### Vacuum Gripper Integration

- Pick force: F = P x A (e.g., 20kPa vacuum on 20mm cup = ~6N)

- Best for: Boxes, sheets, flat objects, electronics

- Not for: Porous, rough, or curved surfaces

- 3D printable: Full ejector and mount systems on Thingiverse

# PART 3: SENSORS

## 3.1 CAMERAS

| Camera | Price | Resolution | Depth? | FOV | Weight | ROS2 Support | Best For |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Raspberry Pi Camera Module 3 | $25-$35 | 12MP (IMX708) | No | 75deg / 120deg | 4g | libcamera + ROS2 | Budget vision, object detection |
| Raspberry Pi Camera v2 | $15-$25 | 8MP (IMX219) | No | 62deg | 3g | libcamera + ROS2 | Even cheaper option |
| Raspberry Pi HQ Camera | $50 | 12MP (IMX477) | No | Lens swap | 50g | libcamera + ROS2 | Machine vision, better quality |
| Arducam OV9281 Global Shutter | $35-$50 | 1MP | No | DFOV 120deg | 5g | ROS2 package | High-speed motion, no rolling shutter |
| Intel RealSense D405 | $240 (refurb $195) | 1280x720 @ 90fps | Yes (IR stereo) | 87deg | 51g | realsense2_camera - mature | Wrist-mounted manipulation (closest range) |
| Intel RealSense D435i | $280 (refurb $225) | 1280x720 @ 90fps | Yes (IR stereo) | 90deg | 72g | realsense2_camera - mature | General SLAM, IMU included |
| Intel RealSense D455 | $320 (refurb $260) | 1280x720 @ 90fps | Yes (IR stereo) | 95deg | 120g | realsense2_camera - mature | Navigation, longer range |
| Orbbec Gemini 335 | $250 | 1280x800 @ 30fps | Yes (active IR) | 91deg | 78g | orbbec-ros2-sdk | Budget depth camera, well supported |
| Orbbec Femto Bolt | $350 | 1024x1024 @ 30fps | Yes (ToF) | 120deg | 178g | orbbec-ros2-sdk | Azure Kinect compatible, large FOV |
| Luxonis OAK-D Pro | $349 | 1280x720 @ 30fps | Yes (neural) | 81deg | 95g | depthai-ros | On-device AI, no GPU needed |
| Stereolabs ZED 2i | $550 | 2208x1242 @ 15fps | Yes (neural) | 110deg | 175g | zed-ros2-wrapper | Outdoor, large FOV, requires CUDA |
| Stereolabs ZED Mini | $450 | 2208x1242 @ 15fps | Yes (neural) | 90deg | 63g | zed-ros2-wrapper | Compact, requires CUDA |
| USB Webcam (Logitech C920) | $30-$50 | 1080p @ 30fps | No | 78deg | 60g | v4l2_camera (ROS2) | Basic vision, teleop |
| Arducam 64MP Hawk-eye | $60-$80 | 64MP | No | 84deg | 20g | libcamera | Ultra-high resolution |

### Camera Notes

- Intel RealSense winding down: No new hardware development. Orbbec is the recommended future-proof alternative.

- Raspberry Pi Camera 3 Wide ($35): Best value for robot navigation - 120deg FOV, autofocus, HDR

- For SLAM: RealSense D435i or Orbbec Gemini 335 (both have built-in IMU for visual-inertial SLAM)

- For manipulation: RealSense D405 (smallest, closest working distance 7cm)

## 3.2 LiDAR

| Model | Price | Range | Scan Rate | Resolution | ROS2 Support | Best For |
| --- | --- | --- | --- | --- | --- | --- |
| YDLIDAR X2 | $78-$90 | 0.12-8m | 5-8Hz | 0.6-0.96deg | ydlidar_ros2_driver | Budget indoor SLAM |
| YDLIDAR X4 | $94-$110 | 0.12-10m | 5-8Hz | 0.43-0.86deg | ydlidar_ros2_driver | Better range budget option |
| YDLIDAR G2 | $178 | 0.12-12m | 6-12Hz | 0.36-0.86deg | ydlidar_ros2_driver | Mid-range indoor |
| YDLIDAR G4 | $334 | 0.12-16m | 5-12Hz | 0.2-0.48deg | ydlidar_ros2_driver | High-res indoor mapping |
| YDLIDAR TG15 (ToF) | $432 | 0.05-15m | 5-12Hz | 0.09-0.22deg | ydlidar_ros2_driver | Outdoor capable, IP65 |
| YDLIDAR Tmini Pro | $144 | 0.02-12m | 6-12Hz | 0.54deg | ydlidar_ros2_driver | Compact ToF option |
| RPLIDAR A1M8 | $65-$100 | 0.15-12m | 5.5Hz | ~0.9deg | rplidar_ros2 | Most popular budget LiDAR |
| RPLIDAR A2M12 | $200-$280 | 0.15-18m | 10Hz | ~0.45deg | rplidar_ros2 | Better scan rate |
| RPLIDAR A3 | $400-$500 | 0.15-25m | 10/20Hz | ~0.33deg | rplidar_ros2 | Higher performance |
| RPLIDAR C1 | $79-$99 | 0.2-12m | 10Hz | ~0.5deg | rplidar_ros2 | Newer ToF version of A1 |
| RPLIDAR S3 | $549 | 0.05-40m | 10-20Hz | ~0.18deg | rplidar_ros2 | Long-range outdoor |
| RPLIDAR S2 | $339 | 0.05-30m | 10Hz | ~0.18deg | rplidar_ros2 | Mid-range outdoor |
| LD06/LD19 (LD LiDAR) | $30-$50 | 0.02-12m | 10Hz | ~0.5deg | Community driver | Ultra-budget option |

### LiDAR Notes

- Best value indoor: RPLIDAR A1 ($65-99) or YDLIDAR X2 ($78) - both work great with ROS2 SLAM Toolbox and Nav2

- Best value outdoor: YDLIDAR TG15 ($432) - IP65 rated, ToF technology

- For SLAM: All listed LiDARs work with slam_toolbox and nav2 in ROS2

- Budget hack: LD06/LD19 ($30-50) - often sold as “D300” on AliExpress, works with ROS2

## 3.3 IMU (Inertial Measurement Unit)

| Sensor | Price | Axes | Fusion | Interface | Accuracy | ROS2 Support | Best For |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MPU6050 | $2-$5 | 6-DOF (accel+gyro) | No | I2C | Moderate | imu_tools | Budget balancing robots |
| MPU9250 | $4-$8 | 9-DOF (+mag) | No | I2C/SPI | Good | imu_tools | Better orientation with magnetometer |
| BNO055 | $8-$15 | 9-DOF | Built-in fusion | I2C/UART | Excellent | bno055 (ROS2) | Best plug-and-play IMU |
| LSM6DS3 | $3-$6 | 6-DOF | No | I2C/SPI | Good | imu_tools | Low power, compact |
| ICM-20948 | $6-$12 | 9-DOF | DMP onboard | I2C/SPI | Very Good | imu_tools | Advanced DMP processing |
| WitMotion WT901B | $20-$35 | 9-DOF | Built-in fusion | UART/I2C | Excellent | UART parser | Rugged, vibration-resistant |
| Phidgets Spatial 3/3/3 | $80-$120 | 9-DOF | Built-in fusion | USB/VINT | Professional | phidgets_ros2 | Professional-grade |

### IMU Notes

- BNO055 is the winner for hobby robotics: Built-in sensor fusion outputs quaternion/orientation directly

- For humanoid balance: MPU6050 + external Kalman filter is sufficient on a budget

- For SLAM: BNO055 or ICM-20948 with imu_filter_madgwick node

- WitMotion units come pre-calibrated and output fused data over UART - very convenient

## 3.4 FORCE / TORQUE SENSORS

| Sensor | Price | Range | Interface | ROS2 Support | Best For |
| --- | --- | --- | --- | --- | --- |
| HX711 + Load Cell | $3-$8 | 1-50kg | GPIO/SPI | Custom node | Grip force, weight measurement |
| Strain Gauge (generic) | $2-$5 | Custom | Wheatstone bridge | ADC + custom | Custom force sensing |
| FSR402 (Force Sensitive Resistor) | $2-$4 | 0.1-10N | Analog | ADC + custom | Touch/pressure detection |
| Mini Load Cell (S-type) | $10-$15 | 1-5kg | HX711 | Custom node | Robot hand grip force |
| Robotis DMS-80 (optional) | $15 | N/A | Analog | Dynamixel SDK | Dynamixel-compatible distance |
| ATI Nano25 | $5,000+ | 25N | Dedicated | Custom | Professional research |
| 6-Axis DIY (Strain gauge + PCB) | $15-$30 | Custom | Custom | Custom | Open source 6-axis F/T sensor |

### Force Sensor Integration Notes

- HX711 is the standard load cell amplifier - works with Arduino/ESP32/Raspberry Pi

- For gripper feedback: FSR402 or mini load cell with HX711 gives basic force feedback

- For arm compliance: 6-axis F/T sensor is ideal but expensive; consider series elastic actuators instead

- ROS2: Write a simple node that publishes geometry_msgs/WrenchStamped from HX711 readings

## 3.5 TOUCH SENSORS

| Sensor | Price | Type | Interface | Best For |
| --- | --- | --- | --- | --- |
| TTP223 Capacitive Touch | $0.50-$1 | Capacitive | Digital GPIO | Touch detection on surface |
| MPR121 | $3-$5 | 12-Channel Cap | I2C | Multi-touch, skin-like sensing |
| Velostat/Linqstat | $2-$5/sheet | Resistive pressure | Analog | Pressure mapping, DIY skin |
| FSR402 | $2-$4 | Force sensitive resistor | Analog | Grip pressure, tactile sensing |
| Piezo element | $0.50-$1 | Vibration/impact | Analog | Collision detection, vibration |
| Conductive thread/fabric | $5-$10 | Resistive | Analog | DIY pressure-sensitive skin |
| Optical (IR reflectance) | $1-$3 | Proximity | Analog/Digital | Near-field object detection |

### Touch Sensor Notes

- DIY robot skin: Velostat sheet + conductive fabric layers + ADC multiplexer

- Capacitive touch: TTP223 modules work through thin plastic/PLA shells (3-5mm)

- Pressure mapping: MPR121 with conductive foam gives 12-point pressure map

- ROS2: Publish sensor_msgs/PointCloud or custom message for tactile array

## 3.6 MICROPHONES / AUDIO

| Component | Price | Interface | ROS2 Support | Best For |
| --- | --- | --- | --- | --- |
| USB Microphone (mini) | $3-$8 | USB | audio_common | Voice commands, basic audio |
| ReSpeaker 2-Mics Pi HAT | $15-$25 | I2S | respeaker_ros | Voice activation, direction |
| ReSpeaker 4-Mic Array | $35-$50 | I2S/USB | respeaker_ros | Direction of arrival, beamforming |
| INMP441 MEMS I2S | $2-$4 | I2S | Custom node | Compact, high quality |
| SPH0645 MEMS I2S | $3-$5 | I2S | Custom node | Raspberry Pi compatible |
| PS3 Eye Camera (mic array) | $5-$10 | USB | audio_common | 4-mic array, ultra cheap |

### Audio Notes

- For voice interaction with Agent 47: ReSpeaker 2-Mic HAT + Raspberry Pi = wake word + command recognition

- For localization: ReSpeaker 4-Mic Array gives sound direction (DOA estimation)

- PS3 Eye Camera is the ultimate audio hack - 4 microphones for $5-10, great for voice recognition experiments

- ROS2: audio_common package handles audio capture and playback

## 3.7 DISTANCE / PROXIMITY SENSORS

| Sensor | Price | Range | Interface | Accuracy | Best For |
| --- | --- | --- | --- | --- | --- |
| HC-SR04 Ultrasonic | $1-$3 | 2-400cm | GPIO trigger/echo | 3mm | Budget obstacle avoidance |
| US-100 Ultrasonic | $2-$4 | 2-450cm | UART/GPIO | 2mm | Better accuracy, UART option |
| JSN-SR04T (waterproof) | $3-$6 | 25-600cm | GPIO | 5mm | Outdoor/moist environments |
| VL53L0X (ToF) | $3-$6 | 2-200cm | I2C | 5mm | Small, accurate indoor ranging |
| VL53L1X (ToF) | $4-$8 | 4-400cm | I2C | 5mm | Longer range ToF |
| VL53L4CD (ToF) | $5-$10 | 1-1300cm | I2C | 10mm | Fast ranging, multi-target |
| TOF10120 | $2-$4 | 10-180cm | I2C/UART | 5mm | Budget ToF option |
| Sharp IR GP2Y0A21YK | $3-$5 | 10-80cm | Analog | 5mm | Simple analog distance |
| Sharp IR GP2Y0A02YK | $4-$6 | 20-150cm | Analog | 10mm | Longer range analog |
| LiDAR TF-Luna | $25-$35 | 0.2-8m | UART/I2C | 1% | Single-point LiDAR |
| LiDAR TF-mini Plus | $35-$45 | 0.1-12m | UART/I2C | 1% | Longer range single-point |

### Distance Sensor Notes

- Best budget indoor: VL53L0X ($3-6) - I2C, accurate, small, multiple can share bus with different addresses

- Best budget outdoor: JSN-SR04T ($3-6) - waterproof ultrasonic

- Obstacle avoidance ring: 6x HC-SR04 ($6-12 total) gives 360-degree coverage

- For navigation: Single-point LiDAR like TF-Luna ($25) is great for height/obstacle detection

# PART 4: ELECTRONICS

## 4.1 MICROCONTROLLERS / COMPUTERS

| Board | Price | CPU | RAM | GPIO | Key Features | ROS2? |
| --- | --- | --- | --- | --- | --- | --- |
| Arduino Nano | $2-$4 | ATmega328P | 2KB | 22 pins | Ultra cheap, simple | Micro-ROS |
| Arduino Mega 2560 | $8-$15 | ATmega2560 | 8KB | 54 pins (15 PWM) | Many IO pins | Micro-ROS |
| ESP32 DevKit V1 | $3-$6 | Dual-core 240MHz | 520KB | 34 pins | WiFi + Bluetooth, dual core | Micro-ROS |
| ESP32-S3 | $4-$8 | Dual-core 240MHz | 512KB | 45 pins | AI acceleration, USB-OTG | Micro-ROS |
| Raspberry Pi Pico | $5 | RP2040 dual-core 133MHz | 264KB | 26 pins | Programmable IO, cheap | Micro-ROS |
| Raspberry Pi Pico 2W | $7 | RP2350 dual-core 150MHz | 520KB | 26 pins | WiFi + BT, very cheap | Micro-ROS |
| Raspberry Pi 4 (4GB) | $55 | BCM2711 quad-core 1.8GHz | 4GB LPDDR4 | 40-pin header | Full Linux, many HATs | Full ROS2 |
| Raspberry Pi 5 (4GB) | $60 | BCM2710 quad-core 2.4GHz | 4GB LPDDR4X | 40-pin header | 2-3x faster than Pi 4 | Full ROS2 |
| Raspberry Pi 5 (8GB) | $80 | BCM2710 quad-core 2.4GHz | 8GB LPDDR4X | 40-pin header | Best for multi-camera | Full ROS2 |
| Jetson Nano 4GB | $149 (hard to find) | Quad-core A57 1.4GHz | 4GB | 40-pin | 128 CUDA cores | Full ROS2 |
| Jetson Orin Nano 4GB | $259 (dev kit) | 6x A78AE 1.5GHz | 4GB LPDDR5 | 40-pin | 20 TOPS AI | Full ROS2 |
| Jetson Orin Nano 8GB | $499 (dev kit) | 6x A78AE 1.5GHz | 8GB LPDDR5 | 40-pin | 40 TOPS AI | Full ROS2 |
| Khadas VIM4 | $200 | A311D2 octa-core | 8GB | 40-pin | Mali GPU, NPU | Full ROS2 |
| LattePanda 3 Delta | $279 | N5105 quad-core | 8GB | 40-pin + Arduino | x86 compatibility | Full ROS2 |

### Microcontroller Notes

- For servo control: Arduino Mega 2560 ($8) can control 48+ servos via 6x PCA9685 boards

- For wireless: ESP32 ($3-6) runs servos + WiFi + Web interface simultaneously

- Main computer: Raspberry Pi 5 (8GB, $80) is the sweet spot for ROS2 + vision

- For AI/vision: Jetson Orin Nano 8GB ($499) if you need on-device neural networks

- Best combo: ESP32 for low-level servo/motor control + Raspberry Pi 5 for high-level ROS2 + vision

## 4.2 MOTOR DRIVERS

### Servo Drivers

| Driver | Price | Channels | Interface | Max Current | Notes |
| --- | --- | --- | --- | --- | --- |
| PCA9685 (AliExpress) | $1.50-$3.00 | 16 | I2C (addr selectable) | 25mA/ch logic | Cascade up to 62 boards = 992 servos |
| PCA9685 (Adafruit) | $14.95 | 16 | I2C | 25mA/ch logic | Quality tested, headers included |
| Servo Driver Shield (Arduino) | $5-$10 | 16 | I2C | 25mA/ch logic | Stacking shield format |
| goBILDA Servo Power Injector | $69.99 | 6 | PWM input | 10A total | High-power servo distribution |
| Dynamixel U2D2 | $49.90 | 253 (Dynamixel) | USB-to-TTL/RS485 | 2A | Required for Dynamixel USB control |
| OpenCM9.04 | $9.90 | 4 servo + 4 Dynamixel | USB/TTL | 2A | ARM Cortex-M3, programmable |

### DC Motor / Stepper Drivers

| Driver | Price | Channels | Voltage | Max Current | Features |
| --- | --- | --- | --- | --- | --- |
| L298N | $2-$4 | 2 DC or 1 stepper | 5-35V | 2A/ch (peak) | Classic, inefficient, cheap |
| TB6612FNG | $3-$6 | 2 DC or 1 stepper | 4.5-13.5V | 1.2A cont/ch | MOSFET, efficient, small |
| DRV8833 | $2-$4 | 2 DC | 2.7-10.8V | 1.2A cont/ch | Compact, good for small motors |
| A4988 | $1-$2.50 | 1 stepper | 8-35V | 1A/ch | Basic stepper, needs heatsink |
| DRV8825 | $2-$4 | 1 stepper | 8-45V | 2.2A/ch | Better than A4988 |
| TMC2209 | $4-$8 | 1 stepper | 4.75-29V | 2.8A/ch | Silent, UART config, stallGuard |
| TMC5160 | $12-$20 | 1 stepper | 8-60V | 4.4A/ch | SPI, highest performance |
| VNH5019 | $8-$12 | 2 DC | 5.5-24V | 12A cont/ch | High current, MOSFET |
| Sabertooth 2x32 | $125 | 2 DC | 6-30V | 32A cont/ch | Professional, RC/serial interface |

### BLDC Drivers

| Driver | Price | Max Power | Voltage | Features |
| --- | --- | --- | --- | --- |
| ODrive S1 | $199 | 1600W | 12-50V | Precision FOC, Web GUI |
| ODrive Pro | $349 | 3000W | 14-58V | Dual encoder, highest precision |
| ODESC 3.6 | $50-$80 | 1000W | 12-50V | ODrive-compatible, lower cost |
| Makerbase ODrive | $30-$50 | 500W | 12-40V | Budget FOC controller |
| VESC 6 MKIII | $150 | 3000W | 14-60V | Excellent for high-power BLDC |
| SimpleFOC Mini | $15-$25 | 200W | 12-30V | Open source FOC, Arduino-based |
| Flipsky FSESC 4.20 | $60 | 1500W | 8-60V | VESC-based, good value |

## 4.3 POWER SYSTEMS

### Batteries

| Battery | Price | Capacity | Voltage | Discharge | Best For |
| --- | --- | --- | --- | --- | --- |
| 2S LiPo 1000mAh | $6-$10 | 1000mAh | 7.4V | 25C | Small robots, Arduino projects |
| 3S LiPo 2200mAh | $12-$20 | 2200mAh | 11.1V | 25C | Standard robot power (Dynamixel, servos) |
| 3S LiPo 5200mAh | $25-$35 | 5200mAh | 11.1V | 30C | Medium robots, longer runtime |
| 4S LiPo 3000mAh | $20-$30 | 3000mAh | 14.8V | 35C | High voltage systems, BLDC |
| 4S LiPo 5000mAh | $30-$45 | 5000mAh | 14.8V | 50C | Large humanoid/quadruped |
| 6S LiPo 5000mAh | $40-$60 | 5000mAh | 22.2V | 50C | ODrive BLDC systems |
| 18650 Li-Ion (pair) | $4-$8 | 2500mAh | 7.4V (2S) | 5-10A | Compact projects, lower current |
| 21700 Li-Ion | $5-$10 | 4800mAh | 3.7V | 15A | Higher capacity, single cell |
| NiMH 12V 3000mAh | $64.99 | 3000mAh | 12V | 10A | goBILDA systems, safer than LiPo |

### Battery Management & Power Distribution

| Component | Price | Function | Notes |
| --- | --- | --- | --- |
| 2S/3S/4S BMS (3-10A) | $2-$5 | Overcharge/overdischarge protection | Essential for LiPo safety |
| 2S/3S/4S BMS (20-30A) | $5-$15 | Higher current protection | For larger robots |
| LiPo Balance Charger (iMAX B6 clone) | $15-$25 | Balance charge 1S-6S | Essential - charge safely |
| LM2596 Buck Converter | $1-$3 | Step-down to 5V/3.3V | 3A max, adjustable |
| XL4015 Buck Converter | $2-$4 | Step-down to 5V/12V | 5A max, more efficient |
| LM2596S with display | $3-$5 | Step-down with voltmeter | Nice for debugging |
| UBEC 5V 3A | $3-$5 | 5V regulated output for servos | From 7.4-26V input |
| UBEC 5V 5A | $5-$8 | Higher current servo power | Needed for many servos |
| BEC 6V 5A | $5-$8 | 6V output (servo voltage) | Direct from LiPo |
| INA219 Current Sensor | $2-$4 | Voltage + current monitoring | I2C interface, power diagnostics |
| Floodgate Power Switch | $34.99 | XT30 switch + current sensing | goBILDA, great for debugging |
| XT60/XT30 connectors | $2-$5/pack | Power connectors | Standard for LiPo batteries |

### Power Budget Examples

| Robot Type | Servo Count | Current Draw | Battery | Runtime |
| --- | --- | --- | --- | --- |
| Small humanoid (12x SG90) | 12 | 2-3A avg | 3S 2200mAh | ~45 min |
| Medium humanoid (16x MG996R) | 16 | 5-8A avg | 3S 5200mAh | ~45 min |
| Large humanoid (16x DS3235) | 16 | 8-12A avg | 4S 5000mAh | ~30 min |
| Quadruped (12x DS3235) | 12 | 6-10A avg | 4S 5000mAh | ~30 min |
| BLDC humanoid (8x ODrive) | 8 BLDC | 20-40A avg | 6S 10000mAh | ~20 min |

## 4.4 COMMUNICATION

| Module | Price | Protocol | Interface | ROS2 Support | Best For |
| --- | --- | --- | --- | --- | --- |
| MCP2515 CAN Module | $1-$3 | CAN 2.0B | SPI | ros2_socketcan | ODrive, CAN servos |
| SN65HVD230 CAN | $3-$5 | CAN 2.0B | 3.3V logic | ros2_socketcan | 3.3V CAN systems |
| TCA9548A I2C Multiplexer | $1-$3 | I2C switch | I2C | Native | Multiple same-address I2C devices |
| PCA9685 (as expander) | $1.50-$3 | PWM | I2C | adafruit_pca9685 | 16-channel PWM over I2C |
| HC-05 Bluetooth | $3-$5 | Bluetooth 2.0 | UART | Custom | Wireless serial, teleop |
| ESP-NOW (ESP32) | $3-$6 | WiFi direct | Native | Custom | Low-latency wireless control |
| nRF24L01+ | $1-$3 | 2.4GHz | SPI | Custom | Very cheap wireless |
| nRF24L01+ PA+LNA | $3-$5 | 2.4GHz + PA | SPI | Custom | Longer range wireless |
| LoRa SX1278 | $5-$10 | 433/868MHz | SPI | Custom | Long range (km), low bandwidth |
| Ethernet W5500 | $3-$5 | Ethernet | SPI | Native | Wired ROS2 networking |
| RS485 Module | $1-$3 | RS485 | UART | Custom | Industrial distance communication |

# PART 5: STRUCTURAL COMPONENTS

## 5.1 ServoCity / Actobotics

| Component | Price Range | Notes |
| --- | --- | --- |
| U-Channel (various lengths) | $3-$8 | Standard 1.5” pattern, aluminum |
| L-Brackets | $2-$5 | Pattern mounts, various angles |
| Ball Bearings | $3-$8 | Flanged, various bore sizes |
| ServoBlocks | $15-$30 | Heavy-duty servo mounts with bearings |
| Servo Shafts / Hubs | $5-$15 | 25T spline adapters |
| Hardware Pack (380pc) | $39.99 | Screws, nuts, standoffs |
| Pattern Plates | $3-$6 | Mounting surfaces |
| Motor Mounts | $3-$8 | NEMA 17, servo, etc. |

### goBILDA (Modern Actobotics Successor)

| Component | Price | Notes |
| --- | --- | --- |
| Mecanum Wheel Set (104mm) | $189.99 | 4x mecanum wheels |
| Strafer Chassis Kit | $699.99 | Complete robot chassis |
| ServoBlock (43mm) | $29.99 | For standard H25T servos |
| 2000 Series Dual Mode Servo | $36.99 | goBILDA branded smart servo |
| FTC Starter Kit 2026-2027 | $899.99 | Full robot build kit |
| USB Camera (Global Shutter) | $74.99 | goBILDA case, MIPI |

## 5.2 3D-PRINTED STRUCTURAL ELEMENTS (On QIDI Plus 4 Max)

Since Nick has a large-format 3D printer, most structural parts can be printed:

| Material | Price/kg | Best For | Print Settings |
| --- | --- | --- | --- |
| PLA+ (eSUN) | $18-$25/kg | Non-structural, prototyping | 200C, 60C bed |
| PETG (eSUN) | $20-$28/kg | Structural parts, impact resistant | 240C, 80C bed |
| ABS (eSUN) | $18-$25/kg | High temp, structural | 250C, 100C bed, enclosure |
| ASA (Polymaker) | $25-$35/kg | UV resistant outdoor parts | 250C, 100C bed, enclosure |
| TPU 95A (eSUN) | $25-$35/kg | Flexures, grippers, bumpers | 220C, 60C bed, slow speed |
| PA6-CF (eSUN) | $35-$50/kg | Maximum strength, lightweight | 260C, 80C bed, hardened nozzle |
| PC (Polymaker) | $30-$40/kg | Extreme strength and temp | 270C, 100C bed, enclosure |

### Recommended Print Strategy for Robot Parts

- Structural frames: PETG or PA6-CF, 4-6 walls, 40%+ infill

- Gearboxes: PA6-CF or PETG, high infill, slow speeds

- Flexure grippers: TPU 95A, 3 walls, 20% infill, 30mm/s

- Servo horns: PA6-CF or ABS, 100% infill (strength critical)

- Covers/cosmetic: PLA+, 2 walls, 15% infill (fast print)

# PART 6: BUDGET TIERS

## TIER 1: $500 Budget - “The Scout”

What Nick can build: A small humanoid (~30cm tall) or a 4-DOF robot arm

| Component | Item | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Brain | Raspberry Pi 5 (4GB) | 1 | $60 | $60 |
| Servos | TowerPro SG90 (arms/gripper) | 8 | $2 | $16 |
| Servos | MG996R (legs/shoulders) | 6 | $5 | $30 |
| Servo Driver | PCA9685 16-ch I2C | 1 | $2 | $2 |
| Camera | Raspberry Pi Camera Module 3 Wide | 1 | $35 | $35 |
| LiDAR | LD06/LD19 (AliExpress) | 1 | $40 | $40 |
| IMU | MPU6050 | 1 | $3 | $3 |
| Microcontroller | ESP32 DevKit | 1 | $4 | $4 |
| Power | 3S LiPo 2200mAh + charger | 1 | $25 | $25 |
| Power | UBEC 5V 5A | 1 | $6 | $6 |
| Power | LM2596 buck converters (5pc) | 1 | $5 | $5 |
| Communication | MCP2515 CAN module | 1 | $2 | $2 |
| Distance | HC-SR04 ultrasonic (5pc) | 1 | $5 | $5 |
| Audio | USB microphone | 1 | $5 | $5 |
| Touch | TTP223 touch sensors (10pc) | 1 | $3 | $3 |
| Current Monitor | INA219 | 1 | $3 | $3 |
| 3D Filament | PETG + TPU (2kg) | 2 | $25 | $50 |
| Screws/hardware | M2/M3/M4 assortment | 1 | $15 | $15 |
| Wires/connectors | Jumper wires, servo cables | 1 | $10 | $10 |
| Gripper | 3D printed SG90 gripper | 1 | $3 | $3 |
|  |  |  | TOTAL | $321 |

Remaining $179: Upgrade to DS3218 servos ($60), add RealSense D405 ($195 - over), or build a second robot

### $500 Build Capability

- 20-30cm tall humanoid, 12-16 DOF

- Camera-based object detection

- 2D LiDAR SLAM navigation

- Basic voice commands

- Gripper pick-and-place of light objects

- Wall-following and obstacle avoidance

## TIER 2: $1,500 Budget - “The Soldier”

What Nick can build: A full-size humanoid (~60cm) or quadruped with proper joints

| Component | Item | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Brain | Raspberry Pi 5 (8GB) | 1 | $80 | $80 |
| AI Accelerator | Raspberry Pi AI Kit (Hailo 8L) | 1 | $70 | $70 |
| Servos - Arms | DS3225 25kg (shoulders/elbows) | 4 | $14 | $56 |
| Servos - Arms | DS3218 20kg (wrists) | 4 | $10 | $40 |
| Servos - Legs | DS3235 35kg (hips/knees) | 6 | $17 | $102 |
| Servos - Head | MG90S (pan/tilt) | 2 | $4 | $8 |
| Servo Drivers | PCA9685 16-ch | 2 | $3 | $6 |
| Camera | Raspberry Pi Camera 3 Wide | 1 | $35 | $35 |
| Depth Camera | Orbbec Gemini 335 | 1 | $250 | $250 |
| LiDAR | RPLIDAR A1M8 | 1 | $80 | $80 |
| IMU | BNO055 | 1 | $12 | $12 |
| Microcontroller | ESP32-S3 | 2 | $6 | $12 |
| Arduino | Arduino Mega 2560 | 1 | $12 | $12 |
| Power | 4S LiPo 5000mAh (x2) + charger | 1 | $80 | $80 |
| Power | UBEC 6V 5A (x3) | 3 | $6 | $18 |
| Power | XL4015 buck converters (5pc) | 1 | $8 | $8 |
| BMS | 4S 30A BMS | 1 | $10 | $10 |
| CAN Bus | MCP2515 modules (x3) | 1 | $6 | $6 |
| Distance | VL53L0X ToF sensors (x6) | 1 | $18 | $18 |
| Audio | ReSpeaker 2-Mic Pi HAT | 1 | $20 | $20 |
| Touch | MPR121 12-ch touch | 2 | $4 | $8 |
| Force | HX711 + load cell (x2) | 1 | $10 | $10 |
| 3D Filament | PETG + TPU + PA6-CF (4kg) | 1 | $100 | $100 |
| Hardware | M2/M3/M4/M5 full assortment | 1 | $30 | $30 |
| Bearings | 608ZZ, 625ZZ assortment | 1 | $20 | $20 |
| Gripper | 3D printed DS3225 adaptive gripper | 1 | $10 | $10 |
|  |  |  | TOTAL | $1,225 |

Remaining $275: Add Dynamixel XL430 for one premium joint, or upgrade to RPLIDAR A2 ($200), or add second depth camera

### $1,500 Build Capability

- 50-70cm tall humanoid, 16-20 DOF with metal gear servos

- Depth camera + LiDAR for full SLAM navigation

- On-device AI object detection (Hailo 8L)

- Voice command + wake word

- Adaptive gripper with force feedback

- Autonomous navigation in mapped environments

- IMU-based balance assistance

- 45+ minute runtime

## TIER 3: $5,000 Budget - “The Commander”

What Nick can build: Professional-grade humanoid or quadruped with Dynamixel actuators

| Component | Item | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Brain | NVIDIA Jetson Orin Nano 8GB | 1 | $499 | $499 |
| Servos - Hips/Knees | XM540-W270-T | 4 | $420 | $1,680 |
| Servos - Shoulders | XM430-W350-T | 4 | $270 | $1,080 |
| Servos - Elbows/Wrists | XL430-W250-T | 4 | $39 | $156 |
| Servos - Head/Hands | XL330-M288-T | 4 | $24 | $96 |
| Dynamixel Controller | OpenCM9.04 + EXP board | 1 | $50 | $50 |
| Dynamixel USB | U2D2 | 1 | $50 | $50 |
| Camera | RealSense D435i (refurb) | 1 | $280 | $280 |
| Camera (wrist) | RealSense D405 (refurb) | 1 | $195 | $195 |
| LiDAR | RPLIDAR A2M12 | 1 | $250 | $250 |
| IMU | BNO055 | 1 | $12 | $12 |
| IMU Premium | Phidgets Spatial 3/3/3 | 1 | $100 | $100 |
| Microcontroller | ESP32-S3 (x2) | 2 | $6 | $12 |
| Power | 3S LiPo 5200mAh (x4) + charger | 1 | $120 | $120 |
| Power | SMPS 12V 20A (Dynamixel) | 1 | $40 | $40 |
| Power | Multiple UBECs and bucks | 1 | $40 | $40 |
| Force/Torque | Custom 6-axis F/T sensor + PCB | 1 | $50 | $50 |
| Touch | MPR121 array (4x) | 1 | $20 | $20 |
| Audio | ReSpeaker 4-Mic Array | 1 | $50 | $50 |
| Distance | VL53L1X array (8x) | 1 | $40 | $40 |
| CAN Bus | Dual CAN + termination | 1 | $20 | $20 |
| Structural | goBILDA bearings, mounts | 1 | $100 | $100 |
| Filament | PA6-CF, PETG, TPU (5kg) | 1 | $150 | $150 |
| Hardware | Full M2-M6 assortment | 1 | $50 | $50 |
| Gripper | 3D printed 3-finger adaptive + FSR | 1 | $30 | $30 |
|  |  |  | TOTAL | $4,810 |

### $5,000 Build Capability

- 70-100cm tall humanoid, 20-24 DOF

- All Dynamixel smart servos with position/velocity/torque feedback

- Dual depth cameras (scene + wrist-mounted)

- High-speed LiDAR SLAM

- 40 TOPS on-device AI (Jetson Orin)

- Full force/torque sensing

- Multi-channel touch sensing

- Directional microphone array

- ~30-45 minute runtime

- Professional ROS2 + MoveIt 2 integration

# PART 7: QUICK REFERENCE TABLES

## Servo Selection Matrix

| Application | SG90 ($2) | MG996R ($5) | DS3218 ($12) | DS3235 ($17) | XL430 ($39) | XM430 ($270) |
| --- | --- | --- | --- | --- | --- | --- |
| Gripper | **** | ** |  |  | * | ** |
| Wrist pan/tilt | **** | *** |  |  | **** | ** |
| Elbow |  | *** | **** | ** | ** | **** |
| Shoulder |  | * | *** | **** | ** | **** |
| Hip |  |  | ** | *** | * | **** |
| Knee |  |  | ** | *** | * | **** |
| Ankle |  |  | *** | **** | ** | **** |
| Quadruped leg |  |  | *** | **** | ** | **** |
| Pan/tilt camera | **** | ** |  |  | *** | * |

## Sensor Priority for Different Robots

| Sensor Type | Humanoid | Quadruped | Arm Only | Mobile Base |
| --- | --- | --- | --- | --- |
| IMU | Essential | Essential | Optional | Helpful |
| Camera (RGB) | Essential | Helpful | Essential | Helpful |
| Depth Camera | Essential | Helpful | Essential | Helpful |
| LiDAR | Essential | Essential | Optional | Essential |
| Force Sensor | Helpful | Optional | Essential | Optional |
| Touch Sensors | Helpful | Optional | Helpful | Optional |
| Microphone | Helpful | Optional | Optional | Optional |
| Ultrasonic/ToF | Helpful | Helpful | Optional | Essential |

## Open Source ROS2 Packages for Integration

| Component | ROS2 Package | Installation |
| --- | --- | --- |
| Dynamixel | dynamixel_sdk, dynamixel_workbench | apt install ros-$ROS_DISTRO-dynamixel-sdk |
| RealSense | realsense2_camera | apt install ros-$ROS_DISTRO-realsense2-camera |
| Orbbec | orbbec_ros2_sdk | GitHub + build from source |
| ZED | zed_ros2_wrapper | Stereolabs repo |
| RPLiDAR | rplidar_ros2 | GitHub + build |
| YDLiDAR | ydlidar_ros2_driver | GitHub + build |
| Camera (generic) | v4l2_camera | apt install ros-$ROS_DISTRO-v4l2-camera |
| IMU (BNO055) | bno055 | apt install ros-$ROS_DISTRO-bno055 |
| IMU (MPU6050) | mpu6050_driver | GitHub + build |
| Servos (PCA9685) | adafruit_pca9685 | pip3 install adafruit-circuitpython-servokit |
| SLAM | slam_toolbox | apt install ros-$ROS_DISTRO-slam-toolbox |
| Navigation | nav2_bringup | apt install ros-$ROS_DISTRO-nav2-bringup |
| Arm Control | moveit2 | apt install ros-$ROS_DISTRO-moveit |
| Audio | audio_common | apt install ros-$ROS_DISTRO-audio-common |

# PART 8: SOURCES & WHERE TO BUY CHEAPEST

## Best Sources by Category

| Category | Cheapest Source | Premium Source |
| --- | --- | --- |
| Servos (cheap) | AliExpress (dsservo official store) | Amazon (faster shipping) |
| Dynamixel | RobotShop (EU), robosavvy (UK) | Robotis official |
| BLDC motors | AliExpress, HobbyKing | ODrive Robotics |
| Motor drivers | AliExpress | Pololu, Adafruit |
| Microcontrollers | AliExpress, LCSC | Adafruit, SparkFun |
| Raspberry Pi | The Pi Hut, Pishop (US) | Official distributors |
| LiPo batteries | AliExpress (Tattu, Gens Ace) | HobbyKing |
| LiDAR | AliExpress (YDLIDAR official) | RobotShop |
| Cameras | AliExpress (Arducam) | Official distributors |
| Structural | AliExpress (brackets) | ServoCity, goBILDA |
| Filament | Amazon (eSUN, Polymaker) | MatterHackers |
| Bearings | AliExpress (bulk) | VXB, McMaster-Carr |
| Sensors (generic) | AliExpress | Adafruit, SparkFun |
| Connectors/wire | AliExpress (bulk) | DigiKey, Mouser |

## AliExpress Store Recommendations

| Store Name | What They Sell | Reliability |
| --- | --- | --- |
| Dsservo Official Store | DS3218/DS3225/DS3235 servos | Excellent |
| RobotDyn Official Store | Motor drivers, sensors, modules | Very Good |
| Waveshare Official | Displays, sensors, HATs | Excellent |
| Landa Tianrui | LiDAR sensors (LD series) | Good |
| YDLIDAR Official Store | YDLIDAR products | Excellent |
| Official Arducam Store | Cameras, lenses | Excellent |
| TZT Official Store | Generic modules, sensors | Good |
| ANGEEK Official Store | Electronics modules | Good |

# SUMMARY: RECOMMENDED FIRST BUILD FOR NICK

## Start Here: $300 “Scout” Mini Humanoid

Given Nick has a QIDI Plus 4 Max (large format, can print big parts), the recommended first build is:

### Bill of Materials (Entry Point)

| Item | Specific Part | Est. Cost |
| --- | --- | --- |
| Main computer | Raspberry Pi 5 (4GB) | $60 |
| Servos (12x) | 8x SG90 + 4x MG996R | $36 |
| Servo driver | PCA9685 16-ch module | $2 |
| Camera | Raspberry Pi Camera 3 Wide | $35 |
| LiDAR | LD06/LD19 | $40 |
| IMU | MPU6050 | $3 |
| Microcontroller | ESP32 DevKit | $4 |
| Battery | 3S LiPo 2200mAh + charger | $25 |
| Power distribution | UBEC 5V 5A + buck converters | $11 |
| Sensors | 4x HC-SR04 + INA219 | $8 |
| Filament | PETG 1kg + TPU 0.5kg | $35 |
| Hardware | Screws, bearings, wires | $25 |
| Audio | USB microphone | $5 |
| TOTAL |  | $289 |

### What This Robot Can Do

- Walk (simple gait, 12 DOF)

- Avoid obstacles (LiDAR + ultrasonic)

- See and recognize objects (Pi Camera + OpenCV)

- Respond to voice commands (Vosk or similar on Pi 5)

- Pick up small objects (SG90 gripper)

- Navigate using SLAM (LiDAR + slam_toolbox)

- Run for ~45 minutes on single charge

- Be fully 3D-printed on QIDI Plus 4 Max

### Next Upgrades (in order)

- $60: Upgrade to DS3218 servos for legs (2x)

- $80: Add BNO055 IMU + WitMotion for better balance

- $250: Add Orbbec Gemini 335 depth camera

- $100: Upgrade to Raspberry Pi 5 (8GB) + AI Kit

- $200: Upgrade to RPLIDAR A1 for better SLAM

- $500+: Start building second unit with Dynamixel XL430

This document compiled from research across AliExpress, Amazon, RobotShop, ServoCity/goBILDA, Adafruit, SparkFun, Dynamixel/ROBOTIS, ODrive Robotics, and academic sources. Prices are estimates in USD as of June 2026 and may vary by vendor and shipping location.

For MEOK Labs internal use. All components verified to have open-source drivers and ROS2 integration.