# Open Source 3D-Printable Humanoid Robot Research for QIDI Plus 4 Max

## Nick at MEOK Labs — Complete Build Guide & Comparison

Research Date: July 2026 Printer: QIDI Plus 4 Max (heated chamber, 350C nozzle, direct drive, multi-material) Materials supported: PLA, PETG, ABS, ASA, PC, PA-CF, PA-GF, TPU

## Table of Contents

- Executive Summary

- Comparison Matrix

- Priority Project: Berkeley Humanoid Lite (Deep Dive)

- InMoov

- Poppy Humanoid

- Reachy Family

- ToddlerBot

- ROBOTO ORIGIN

- AgiBot Lingxi X1

- pib (Printable Intelligent Bot)

- K-Bot (K-Scale Labs)

- NAO (SoftBank/Aldebaran)

- Fourier GR-1

- Unitree G1

- EngineAI PM01

- LeRobot SO-100 / SO-101 Arms

- Apptronik Apollo

- UBTech Walker Series

- Tesla Optimus

- Other Notable Projects

- Best First Print Order

- QIDI Plus 4 Max Recommended Settings

- Resources & Links

## Executive Summary

This document catalogs every major open source humanoid robot project that can be built with a 3D printer, with special focus on the QIDI Plus 4 Max heated-chamber printer. Nick at MEOK Labs has an ideal printer for this work — the heated chamber (up to 350C nozzle) enables printing not just PLA/PETG but also PA-CF, PC, and ABS for structural robot parts.

Top 5 recommendations for Nick:

| Rank | Project | Why |
| --- | --- | --- |
| 1 | Berkeley Humanoid Lite | The best balance of open-source, cost (~$5k), active community, proven walking, designed for FDM printing |
| 2 | ROBOTO ORIGIN | Full-stack open source, runs 3m/s, ~$6.8k BOM, very active 2026 project |
| 3 | ToddlerBot | Stanford-backed, ML-first, compact, 30 DOF, under $6k |
| 4 | InMoov | Classic proven platform, cheapest entry point (~$800), massive community |
| 5 | K-Bot | Open source, $8,999, full-size, YC-backed (note: company shut down, IP released) |

## Comparison Matrix

| Project | License | BOM Cost | Height | Weight | DOF | Print Material | QIDI Compatible | Difficulty | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Berkeley Humanoid Lite | MIT (code) / CC-BY-SA 4.0 (HW) | ~$5,000 | 0.8m | 16kg | 22 | PLA (std) | YES | Advanced | Active, buildable NOW |
| InMoov | GPL/open | $800-$2,500 | 1.8m | ~30kg | 22-45 | PLA/PETG | YES | Intermediate | Mature, proven |
| Poppy Humanoid | CC-BY-SA | $8,000-$9,000 | 84cm | ~3.5kg | 25 | PLA/ABS | YES | Advanced | Mature |
| Reachy Mini | Apache 2.0 | $299 | 28cm | 1.5kg | 6 | (pre-built) | N/A | Beginner | Pre-order |
| Reachy 2 | Apache 2.0 | ~$70,000 | ~1.4m | ~20kg | ~17 | (pre-built) | N/A | Advanced | Available |
| ToddlerBot | MIT (code) / CC-BY-NC (HW) | ~$6,000 | 0.56m | 3.4kg | 30 | PLA/PETG | YES | Advanced | Active, 2.0 released |
| ROBOTO ORIGIN | GPL v3 | ~$6,800 | 1.25m | 34kg | 23 | Mixed | YES | Advanced | Very active 2026 |
| AgiBot Lingxi X1 | Open (GitHub) | ~$15,000? | 1.33m | 33kg | 34 | Mixed | Partial | Expert | Open sourced 2024 |
| pib | Open source | ~$500-800 | 89cm (upper) | ~5kg | 16+ | PLA/PETG | YES | Beginner | Active, v4 released |
| K-Bot | CERN-OHL-S / GPL v3 | $8,999 kit | 1.4m | 35kg | Full | Mixed | Partial | Expert | Company shut down |
| NAO v6 | Proprietary + SDK | ~$7,500 | 58cm | 5.4kg | 25 | (pre-built) | N/A | Beginner (use) | Available |
| Fourier GR-1 | Closed (some SDK) | ~$125,000 | 1.65m | 55kg | 25-40 | (pre-built) | N/A | Expert | Available |
| Unitree G1 | Closed (SDK) | $16,000 | 1.32m | 35kg | 23-43 | (pre-built) | N/A | Expert | Available |
| EngineAI PM01 | Open SW, closed HW | ~$12,000 | 1.38m | 40kg | 24 | (pre-built) | N/A | Advanced | Available |
| SO-100 Arm | Apache 2.0 | ~$100-200 | N/A | ~0.5kg | 6 | PLA/PETG | YES | Beginner | Very active |
| Apptronik Apollo | Closed | ~$50-300k | 1.73m | 73kg | 44 | (pre-built) | N/A | Expert | Available |
| UBTech Walker S | Closed (SDK) | ~$50-150k | 1.7m | 76kg | 41 | (pre-built) | N/A | Expert | Available |
| Tesla Optimus | Closed (patents) | ~$20k (target) | 1.73m | ~57kg | 28+ | (pre-built) | N/A | N/A | In development |

## 1. Berkeley Humanoid Lite (PRIORITY PROJECT)

### Overview

Berkeley Humanoid Lite is an open-source, sub-$5,000 humanoid robot developed at UC Berkeley, featuring modular 3D-printed cycloidal gearboxes and widely available components. Released in 2025, it’s the most distinctive open-hardware humanoid target available right now.

- Paper: arXiv:2504.17249 (presented at RSS 2025)

- Status: Buildable NOW — v1.1.0 released Sept 2025, community actively building

- Height: 0.8m (80cm) — resembles a 5-year-old child

- Weight: 16kg

- DOF: 22 (6 left arm, 6 right arm, 5 left leg, 5 right leg)

- Walking: Yes — RL-based policies with sim-to-real transfer

### License

- Code: MIT License

- Hardware assets (CAD, STLs, docs): Creative Commons Attribution-ShareAlike 4.0 (CC-BY-SA 4.0)

- This is a genuinely open, commercially-usable license. You CAN sell derivatives.

### Full BOM (~$5,000)

| Category | Component | Qty | Est. Cost (US) | Est. Cost (China) | Source |
| --- | --- | --- | --- | --- | --- |
| Actuators | 6512 Actuator (larger, for legs) | 8 | ~$120 each | ~$90 each | Self-built |
| Actuators | 5010 Actuator (smaller, for arms) | 10 | ~$100 each | ~$75 each | Self-built |
| Motor | M6C12 150KV BLDC drone motor (MAD Components) | 18 | $84 | $62 | AliExpress |
| Motor Driver | B-G431B-ESC1 (STM32-based) | 18 | ~$25 | ~$18 | Mouser/DigiKey |
| Bearings | 6811ZZ ball bearing | 18 | ~$5 | ~$3 | AliExpress |
| Frame | 2020 Aluminum extrusion | 1 set | ~$30 | ~$20 | Amazon/AliExpress |
| Computer | Intel N95 Mini PC | 1 | ~$120 | ~$100 | Amazon/AliExpress |
| IMU | Cell-phone grade IMU (over Arduino) | 1 | ~$25 | ~$15 | Amazon |
| USB-CAN | USB to CAN adapter | 4 | ~$15 | ~$10 | Amazon |
| Arduino | Arduino (for IMU interface) | 1 | ~$10 | ~$5 | Amazon |
| Battery | 6S 22.2V 4000mAh LiPo | 1-2 | ~$60 | ~$45 | HobbyKing |
| Power mgmt | Power distribution board | 1 | ~$30 | ~$20 | Self-built or bought |
| Screws/fasteners | M3, M4, M5 hardware kit | 1 | ~$30 | ~$20 | Amazon |
| Wiring | CAN cables, power cables | 1 set | ~$30 | ~$20 | Amazon |
| Joystick | Logitech F710 (for control) | 1 | ~$40 | ~$30 | Amazon |
| 3D Filament | PLA (2-3kg total) | 3kg | ~$60 | ~$45 | Bambu/Hatchbox |
| Optional | SteamVR for teleop | 1 set | ~$200+ | ~$200+ | Valve/HTC |
| TOTAL |  |  | ~$4,500-5,500 | ~$3,500-4,500 |  |

Notes on BOM: - The 6512 and 5010 actuators are the SAME modular design — different sizes for different torque requirements - All actuator housings, cycloidal gears, input shafts, and output shafts are 3D printed - A brass hex stand is embedded in the input shaft for stiffness - Full BOM spreadsheet with links: https://docs.google.com/spreadsheets/d/1AQEHcH_nPkXYfor2-h7bwNIUMmsePtAm53epnsWgZXc/edit

### Actuator Details (Key Innovation)

The cycloidal gearbox design is the key innovation: - Large gear teeth distribute load across a larger surface area than traditional gears - This compensates for PLA’s inherent weakness vs. CNC aluminum - Two actuator sizes: 6512 (65mm diameter, 12:1 reduction) and 5010 (50mm, 10:1) - Single encoder on motor shaft (no joint encoder) — requires calibration after each power cycle

### 3D Print Settings

Critical: Designed for Bambu Lab X1C but works on QIDI Plus 4 Max

| Setting | Actuator Housing | Actuator Shaft | Robot Body Parts |
| --- | --- | --- | --- |
| Material | PLA (recommended) | PLA (recommended) | PLA (recommended) |
| Layer height | 0.2mm | 0.2mm | 0.2mm |
| Wall loops | 4+ | 4+ | 3+ |
| Top/bottom layers | 5+ | 5+ | 4+ |
| Infill | 40%+ Gyroid | 40%+ Gyroid | 25%+ Gyroid |
| Print orientation | Flat (follow docs) | Flat (follow docs) | As oriented in project |
| Support | Tree support | Tree support | As needed |
| Brim | Recommended for adhesion | Recommended | Optional |

For QIDI Plus 4 Max specifically: - Use the heated chamber for better layer adhesion - PA-CF or PC could be used for high-stress parts (gearbox housings) for extra durability - However, the design is validated for PLA — don’t overcomplicate initially - Mirror parts along X axis for left/right limbs

### Electronics Architecture

- Intel N95 Mini PC — runs RL policies + low-level control

- CAN 2.0 bus at 1 Mbps — 4 separate CAN buses (one per limb)

- USB-to-CAN adapters — interface between PC and motor drivers

- B-G431B-ESC1 — STM32-based motor controller running custom firmware

- Arduino — reads IMU over USB

- Communication rate: 250 Hz to actuators and IMU

- Power: 6S LiPo, ~30 min runtime

### Assembly

- Estimated time: ~3 days for experienced builder, ~1 week for novice

- Assembly guide: https://berkeley-humanoid-lite.gitbook.io/docs/

- Tools needed: Standard metric hex keys, soldering iron, multimeter, crimping tool

- Key steps: Print actuators first → Assemble and test each actuator → Flash motor firmware → Calibrate joints → Build torso frame → Mount PC and electronics → Attach limbs → Test walking

### Software Stack

- Simulation: Isaac Lab (NVIDIA) — RL training

- Low-level control: Custom C controller running on the N95 PC

- Policy deployment: RL-trained policies transfer zero-shot from sim to real

- Teleoperation: SteamVR (optional, for manipulation tasks)

- ROS2: Compatible (URDF/MJCF/USD files provided)

- LeRobot: Integration available (Hugging Face)

### Community & Resources

- GitHub: https://github.com/hybridrobotics/berkeley-humanoid-lite

- Documentation: https://berkeley-humanoid-lite.gitbook.io/docs/

- Discord: Active community (linked from GitHub)

- CAD: Onshape (editable) + STL releases on GitHub

- Paper: arXiv:2504.17249

- Video: Multiple build and walking videos on project page

- MakerWorld: Community 3D print files shared

### Known Issues & Workarounds

- Joint calibration required after each power cycle (no absolute encoders) — automated via provided script

- PLA gears may wear over time — print spares, consider PA-CF for gears on QIDI Plus 4 Max

- CAN bus wiring requires care — follow wiring guide closely

- Initial firmware flashing requires STM32 tools

## 2. InMoov

### Overview

The original and most iconic open-source 3D-printed humanoid robot. Created by French sculptor Ga”el Langevin in 2012. It’s a life-size, modular humanoid with the largest community in the world.

- Creator: Ga”el Langevin (France)

- Height: Life-size (~1.75-1.8m depending on build)

- Weight: ~30kg

- DOF: 22-45 depending on configuration (head: 6, torso: 2, arms: 10, hands: 16, legs: non-motorized by default)

- Community: 10,000+ builders worldwide

### License

- Fully open source — all CAD, STLs, electronics schematics, and code freely available

- Effectively public domain / GPL-like (no formal single license)

- Can be used commercially

### BOM (~$800-$2,500)

| Component | Details | Cost |
| --- | --- | --- |
| Servo motors (28x) | MG996R, HS-805BB, DS3218, etc. | $200-400 |
| Arduino Mega 2560 | 2 units | $30-50 |
| Nervo Board shields | Custom shield for Arduino | $30-60 (or self-built) |
| Cameras | 2x USB webcams for eyes | $20-40 |
| Kinect sensor | For 3D depth/gesture | $30-50 (used) |
| PIR sensor | Presence detection | $3-5 |
| Speakers | 2x for speech | $10-20 |
| Power supply | 5V/6V bench supply | $30-50 |
| Filament (PLA) | ~5-8kg total | $60-120 |
| Hardware kit | Screws, bearings, cables | $50-100 |
| Electronics misc | Wires, connectors, sensors | $50-100 |
| TOTAL |  | $800-2,500 |

### 3D Printability

- Build volume required: 12x12x12cm minimum (most parts fit on QIDI Plus 4 Max)

- Material: PLA recommended (parts designed for PLA)

- Number of parts: ~57 different unique parts, ~200+ total pieces

- Print time: 200-400 hours depending on settings

- Best first print: Start with fingers, then hand, then forearm

### Electronics

- 2x Arduino Mega 2560 — one for body, one for head

- Nervo Boards — custom servo driver shields

- MyRobotLab — open-source Java-based control software

- ROS: Community ports available

- Speech recognition: Supports multiple engines (Google, local)

- Vision: OpenCV integration

### Software

- MyRobotLab (primary): https://myrobotlab.org/

- Language support: Python, Java

- Features: Speech recognition, object tracking, gesture recognition, chatbot, weather

- VR teleoperation: Community extensions

### Assembly Difficulty

- Time: 40-80 hours of assembly

- Skills: Basic soldering, 3D printing, Arduino programming

- Approach: Modular — build body sections independently (hand, arm, head, torso)

### Resources

- Website: https://inmoov.fr/

- Thingiverse: https://www.thingiverse.com/inmoov

- GitHub: Multiple community repos

- MyRobotLab: https://myrobotlab.org/

- YouTube: Ga”el Langevin channel has extensive build videos

- Community: MyRobotLab forums, Discord

## 3. Poppy Humanoid

### Overview

Open-source 3D-printed humanoid from Inria Bordeaux (France). Research and education focused. Uses high-quality Robotis Dynamixel actuators.

- Origin: Flowers laboratory at Inria Bordeaux, 2012

- Managed by: Poppy Station (non-profit)

- Height: 84cm

- Weight: ~3.5kg

- DOF: 25 (powered by Dynamixel servos)

### License

- Hardware: CC-BY-SA (Creative Commons Attribution-ShareAlike)

- Software: GPL v3

- Name “Poppy” is trademarked — you can build/modify but can’t call commercial products “Poppy”

### BOM (~$8,000-$9,000)

- 25x Robotis Dynamixel actuators (~$200 each = $5,000)

- XL-320 (smaller) and MX-28 (larger) Dynamixel servos

- Raspberry Pi 3/4 as main computer

- 3D printed parts (STL files on GitHub releases)

- Power supply, wiring, sensors

- Assembly time: ~7 hours for experienced builder

### Key Features

- Snap! visual programming — beginner-friendly

- Python API — advanced control

- Pypot library — motor control and kinematics

- Simulation: V-REP integration

- Designed for: Research in education, AI, robotics

### Resources

- GitHub: https://github.com/poppy-project/poppy-humanoid

- BOM: https://github.com/poppy-project/poppy-humanoid/blob/master/hardware/BOM.md

- Assembly: https://github.com/poppy-project/poppy-humanoid/blob/master/hardware/doc/Poppy_Humanoid_assembly_instructions.md

- STL files: Available in GitHub releases

- Forum: https://forum.poppy-project.org/

- Full kits: Available from G”en”eration Robots (Europe)

### QIDI Compatibility

- YES — all parts designed for standard desktop FDM printers

- Material: PLA or ABS recommended

- Build volume: Standard 12x12x12cm sufficient

## 4. Reachy Family

### Reachy 2 (Full-Size Humanoid)

- Price: ~$70,000 (email sales)

- License: Apache 2.0 (SDK and software)

- Height: ~1.4m

- Weight: ~20kg

- DOF: 17 (7 per arm + 3 for head/neck)

- Status: Available now, used in Cornell, Carnegie Mellon labs

- GitHub: https://github.com/pollen-robotics

### Reachy Mini (Desktop Robot) — 2025

- Price: Starting at $299

- License: Fully open source (hardware CAD + Python SDK)

- Height: 28cm

- Weight: 1.5kg

- DOF: 6 (head movement)

- Features: Expressive LED eyes, head tracking, 360-degree body rotation

- Versions: Lite (USB) and Wireless (Raspberry Pi CM4 + WiFi)

- GitHub: https://github.com/pollen-robotics/reachy_mini

- Hugging Face: Integrated with HF Spaces app store

- Status: Pre-order, shipping late 2025

### Reachy 2019 (Original)

- License: Apache 2.0

- Software: pip install reachy-2019

- 3D models and code: Full open source

- Documentation: https://pollen-robotics.github.io/reachy-2019-docs/

### Key Notes

- Pollen Robotics was acquired by Hugging Face in April 2025

- Strong integration with LeRobot ecosystem

- The Amazing Hand — fully 3D printed robotic hand for <$200

## 5. ToddlerBot

### Overview

Stanford University’s open-source humanoid robot designed for ML policy learning. Extremely compact and capable. One of the most ML-compatible platforms available.

- Institution: Stanford University (REALab + TML)

- Paper: arXiv:2502.00893 (CoRL 2025)

- Height: 0.56m (56cm)

- Weight: 3.4kg

- DOF: 30 (7 per arm, 6 per leg, 2 neck, 2 waist)

- BOM cost: Under $6,000

### License

- Code: MIT License

- Design (CAD/STL): CC-BY-NC (non-commercial) — IMPORTANT for commercial use

- Onshape CAD: Available and editable

### Key Features

- Cartwheel capable — highly dynamic movement

- Crawling — uses arms and legs coordinated

- VR teleoperation — Meta Quest 2

- Omnidirectional walking — RL with zero-shot sim-to-real

- Bimanual manipulation — RGB diffusion policy

- Payload: 1.48kg (40% of body weight)

- Runtime: 19 minutes continuous walking

- Repair time: 21 min printing + 14 min assembly for full restoration

### Actuators

- ROBOTIS Dynamixel: 2XC430, 2XL430, XC330, XC430, XM430

- U2D2 communication adapter

- Reference: ROBOTIS sells a ToddlerBot bundle kit

### Electronics

- Jetson Orin NX 16GB — onboard computing

- 2x fisheye cameras — stereo vision

- 2x microphones + speaker

- IMU

### Resources

- GitHub: https://github.com/hshi74/toddlerbot

- Website: https://toddlerbot.github.io/

- CAD: Onshape (linked from GitHub)

- 3D files: MakerWorld (Bambu Lab)

- Community: Discord + WeChat

- BOM: https://hshi74.github.io/toddlerbot/hardware/01_bill_of_materials.html

### QIDI Compatibility

- YES — all parts 3D printed, designed for FDM

- Material: PLA or PETG recommended

- Build volume of QIDI Plus 4 Max more than sufficient

## 6. ROBOTO ORIGIN

### Overview

The world’s first full-stack open-source bipedal humanoid robot, developed by Xiaomi-backed RoboParty in just 120 days. One of the most exciting new projects of 2026.

- Company: RoboParty (Shanghai), founded Feb 2025 by 21-year-old Yi Huang

- Funding: $10M seed from Xiaomi, MPCi, Galbot

- Height: 1.25m

- Weight: 34kg

- DOF: 23

- BOM cost: ~49,743 CNY (~$6,800 USD)

- Max speed: 3 m/s (running!)

- GitHub stars: 1,000+ in first month

### License

- Software: GPL v3

- Hardware: Full open source (structural drawings, EBOM, SOPs, supplier lists)

- All sub-repositories have specific licenses

### Full Stack Open Sourced

| Module | Repository | Content |
| --- | --- | --- |
| Hardware | rpo_hardware | CAD, PCB, BOM, structural drawings |
| Deployment | roboparty_deploy | ROS2 middleware, control |
| Training | roboparty_train | RL in IsaacLab, Sim2Sim |
| Description | rpo_description | URDF/MJCF kinematic models |
| Firmware | roboparty_firmware | USB2CAN, OrangePi build |
| Navigation | roboparty_navigation | Navigation modules |
| XR Teleop | roboparty_xr_teleop | VR teleoperation |

### Key Specs

- Actuators: DM 4340P and DM 10010L servo actuators

- Computer: Orange Pi 5 Plus (RK3588)

- IMU: HIPNUC HI13

- Battery: 48V 15Ah lithium-ion

- Software: ROS2 deployment framework, IsaacLab training

- Gait: AMP (Adversarial Motion Priors) for natural walking

- Motion: SMPL-X human model adaptation

### Resources

- GitHub: https://github.com/Roboparty/roboto_origin

- Sub-repos: 8 separate repositories (see table above)

- Pre-orders: ~100 development kit pre-orders

- Community: QQ Group: 546376843

### QIDI Compatibility

- YES — designed for DIY assembly with FDM printing

- Components sourced from Taobao + rapid prototyping services

## 7. AgiBot Lingxi X1

### Overview

Fully open-source humanoid robot from Chinese robotics company AgiBot (Zhiyuan Innovation), founded by former Huawei “Genius Youth” Peng Zhihui. One of the most significant open-source releases from China.

- Company: AgiBot (Shanghai), founded Feb 2023

- Open sourced: October 24, 2024

- Height: 1.33m (133cm)

- Weight: 33kg

- DOF: 34

- BOM cost: Estimated ~$15,000 (with proprietary PowerFlow servos)

### License

- Full open source — hardware blueprints + software code

- Total data size: 1.2 GB

- Includes: structural drawings, hardware block diagrams, full BOM (every screw and gear), assembly instructions

- Software: AimRT framework, URDF, Sim2Sim, RL training code, inference tools

### Key Components

- PowerFlow servos — proprietary AgiBot actuators (high torque density)

- AimRT framework — open-source robotics middleware

- Sensors: RGBD cameras, LiDAR, force sensors, IMU

- Battery: ~2 hours

### Important Note

- The PowerFlow servos are proprietary — you can’t easily source them independently

- This makes self-building more challenging than Berkeley Humanoid Lite

- Better as a reference design than a DIY build

### Resources

- GitHub: https://github.com/AgibotTech (multiple repos)

- Training: https://github.com/AgibotTech/agibot_x1_train

- Inference: https://github.com/AgibotTech/agibot_x1_infer

- AimRT: https://www.aimrt.org/

- Website: https://www.agibot.com/

## 8. pib (Printable Intelligent Bot)

### Overview

German open-source 3D-printable humanoid robot from isento GmbH. Focused on education and makers. Now in version 4.

- Company: isento GmbH, Nuremberg, Germany

- Height: 89cm (upper body), 68cm arms

- Weight: ~5kg

- Community: 1,800+ members

- Used in: 70+ schools and educational institutions

### License

- Open source — STL files, assembly instructions, code freely available

- Awards: German Design Award 2025, German Innovation Award 2025

### Key Features (v4)

- Microphone array — detects sound direction

- New speakers with blue accent lighting

- 3 programmable RGB buttons on torso

- Simplified design — easier motor calibration, better electronics access

- AI integration — ChatGPT and other LLMs

### BOM

- Cost: ~$500-800 for full build

- Servos: Standard hobby servos (MG996R or similar)

- Electronics: Arduino/Raspberry Pi based

- Filament: ~2-3kg PLA or PETG

- Shop available: Complete component sets sold on pib.rocks

### Resources

- Website: https://pib.rocks/

- Shop: https://pib.rocks/shop/

- Community: Discord server

- Education platform: pib.Education

### QIDI Compatibility

- YES — designed for standard desktop FDM printers

- Material: PLA or PETG

- Difficulty: Beginner to intermediate

## 9. K-Bot (K-Scale Labs)

### Overview

Open-source full-size humanoid robot from Y Combinator startup K-Scale Labs. The company shut down in late 2024 but released all IP under open licenses.

- Company: K-Scale Labs (Palo Alto), YC W24 — SHUT DOWN

- Final price: $8,999 (Founder’s Edition)

- Height: 1.4m

- Weight: 35kg

- License: CERN-OHL-S (hardware) / GPL v3 (software)

### Key Details

- Company shut down due to funding issues (couldn’t raise Series A)

- All proprietary IP released under open licenses

- Over $2M in pre-orders at time of shutdown

- K-Bot was shipping to customers before shutdown

### Resources

- GitHub: https://github.com/kscalelabs/kbot

- Docs: https://docs.kscale.dev/

- Status: Community can continue development

- Philosophy: “Android for the real world”

### Important Caveat

- Company no longer exists — no support, no warranty

- But the open-source community can fork and continue

- Good for experienced builders who can self-support

## 10. NAO (SoftBank / Aldebaran)

### Overview

The classic educational humanoid robot. Not 3D-printable but has significant open-source SDK elements. NAOqi OS is accessible for development.

- Manufacturer: Aldebaran Robotics (now SoftBank Robotics Europe)

- Height: 58cm

- Weight: 5.4kg

- DOF: 25

- Price: ~$7,500 (V6)

- Units deployed: 13,000+ in 70+ countries

### Open Elements

- NAOqi SDK — Python, C++, Java APIs

- Choregraphe — visual programming tool

- ROS integration — nao_bringup package

- Simulation — Webots integration

- URDF models — Available for simulation

### Limitations

- Hardware is NOT open source — cannot 3D print or modify hardware

- SoftBank Robotics Europe entered receivership in 2025 — future uncertain

- Primarily a closed platform with open SDK

### Resources

- Documentation: http://doc.aldebaran.com/

- Community: ROS NAO packages, various forums

- GitHub: https://github.com/cyberbotics/naoqisim (simulator)

## 11. Fourier GR-1

### Overview

Advanced humanoid from Shanghai-based Fourier Intelligence. NOT open source hardware, but some open software elements.

- Price: ~$125,000

- Height: 1.65m

- Weight: 55kg

- DOF: 25-40 (depending on configuration)

- Speed: 5 km/h walking, 50kg payload

- Status: Available, mass-produced

### Open Elements

- NVIDIA Isaac Gym integration for RL training

- Some SDK access

- NOT a DIY platform — fully manufactured product

### Resources

- Website: https://fourierintelligence.com/gr1/

- NVIDIA blog: https://developer.nvidia.com/blog/spotlight-fourier-trains-humanoid-robots-for-real-world-roles-using-nvidia-isaac-gym/

## 12. Unitree G1

### Overview

$16,000 commercial humanoid from Unitree (known for quadrupeds). NOT open source, but popular platform.

- Price: $16,000 (standard), ~$25,000 (EDU)

- Height: 1.32m

- Weight: 35kg

- DOF: 23-43 (depending on version)

- Speed: Various demos show impressive mobility

- Open elements: SDK + ROS2 support, but NOT open hardware

### Can You Print a Clone?

- No complete open-source clone exists

- Some community projects attempt to replicate the form factor

- The Unitree actuators are proprietary and not available separately

- Better to build Berkeley Humanoid Lite or ROBOTO ORIGIN for a similar form factor

### Resources

- Website: https://www.unitree.com/products/g1

- GitHub: Unitree provides SDK repos

## 13. EngineAI PM01

### Overview

Compact open-software humanoid from Shenzhen-based EngineAI. Open software, closed hardware.

- Price: $12,000-$25,500

- Height: 1.38m

- Weight: 40kg

- DOF: 24

- Speed: 2 m/s (can run!)

- Did front flip: World first for compact humanoid

### Open Elements

- Open-source software: Control algorithms on GitHub

- RL training framework: Based on MIT Cheetah Software

- Simulation: engineai_legged_gym (RL training)

- Hardware: NOT open — pre-built robot only

### Key Specs

- Compute: Intel N97 CPU + NVIDIA Jetson Orin

- Camera: Intel RealSense depth camera

- Battery: 2 hours, swappable

- Waist: 320-degree rotation

### Resources

- GitHub: https://github.com/engineai-robotics/engineai_humanoid

- RL training: https://github.com/engineai-robotics/engineai_legged_gym

- Website: https://en.engineai.com.cn/

## 14. LeRobot SO-100 / SO-101

### Overview

While not a full humanoid, the SO-100/SO-101 arms are the most accessible entry point into the Hugging Face LeRobot ecosystem. Start here if you want to learn before building a full humanoid.

- SO-100 price: ~$100

- SO-101 price: ~$150-200

- DOF: 6 per arm

- License: Apache 2.0

### Key Features

- 3D printed — all structural parts

- FeeTech STS3215 servos — $10-15 each

- LeRobot integration — train policies on real hardware

- Teleoperation: Leader-follower setup (SO-101 supports phone-based teleop)

- GR00T fine-tuning — NVIDIA’s humanoid model runs on SO-100

- Bimanual setup — use two arms together

### Resources

- BOM + Assembly: https://github.com/TheRobotStudio/SO-ARM100

- LeRobot docs: https://huggingface.co/docs/lerobot

- Community: 5,000+ datasets shared on Hugging Face Hub

### QIDI Compatibility

- YES — all parts PLA/PETG, standard FDM

- Perfect first project for Nick before attempting full humanoid

## 15. Apptronik Apollo

### Overview

Commercial humanoid designed for industrial work. Closed source.

- Price: $50,000-$300,000

- Height: 1.73m

- Weight: 73kg

- DOF: 44

- Runtime: 4 hours (hot-swappable battery)

- Payload: 25kg per arm

### Open Elements

- NONE — fully proprietary

- Raised $350M from Google and others

- Uses proprietary linear actuators

### Verdict

Not a DIY platform. Reference only.

## 16. UBTech Walker Series

### Overview

Chinese humanoid robots from publicly-listed UBTech. Closed source with some open interfaces.

- Models: Walker C, S, S2, X, TG Walker

- Height: 1.63-1.7m

- DOF: 20-52

- Price: ~$50,000-$300,000

- Features: Factory deployment (NIO, BYD, Foxconn)

### Open Elements

- TG Walker (research version): Open interfaces for joints and sensors

- URDF models: Available for simulation

- RL training frameworks: Open-source components

- Datasets: To be released progressively

- ROSA 2.0: Integrates with ROS2

### Verdict

Not a 3D-printable platform. Reference for commercial state-of-the-art.

## 17. Tesla Optimus

### Overview

Tesla’s humanoid robot. Closed source with patent publications.

- Target price: $20,000-$30,000

- Height: 1.73m

- Weight: ~57kg

- DOF: 28+

- Status: In development, summer 2026 low-volume production target

### Open Elements

- Patents only — published April 2026 revealing hand architecture

- 25-actuator, 22-DoF hand design detailed

- NOT open source — patents are defensive, not permissive

### Verdict

Not a DIY platform. Reference only.

## 18. Other Notable Projects

### Booster Robotics T1 / K1 Geek

- Company: Booster Robotics (China)

- K1 Geek: 95cm, 19.5kg, 22 DOF, $5,999

- RoboCup 2025 KidSize champion

- Open source: Development platform, ROS2 support

- Website: https://www.boostertech.co/

### Hope-JR Humanoid Arm

- Integrated with LeRobot

- Open source arm design

- GitHub: https://github.com/TheRobotStudio/HopeJr

### dmBot

- Height: 0.76m, 15kg, 18 DOF

- BOM: ~$3,000

- Open source: Yes (per comparison table in Berkeley paper)

### Noetix N2

- Height: 1.18m, 30kg, 18 DOF

- Company: Noetix Robotics

- Status: Available

## Best First Print Order for Nick at MEOK Labs

### Phase 1: Build Skills (Week 1-2)

- Print SO-100 arm (~$100, 20-30 hrs printing)

  - Learn LeRobot ecosystem

  - Understand servo control, teleoperation

  - Get familiar with policy training

### Phase 2: Validate Printer for Humanoid Parts (Week 2-3)

- Print one Berkeley Humanoid Lite actuator

  - Print the 5010 actuator housing + shaft

  - Test fit with bearings and motor

  - Validate your print settings

  - This is the CORE building block — master this first

### Phase 3: Full Actuator Production (Week 3-5)

- Print all 18 actuator housings (10x 5010, 8x 6512)

- Print all cycloidal gears and shafts

- Assemble and test each actuator individually

### Phase 4: Frame and Electronics (Week 5-7)

- Print torso frame parts

- Print leg and arm structural parts

- Cut aluminum extrusion

- Flash motor firmware to all 18 ESCs

### Phase 5: Integration (Week 7-9)

- Mount all actuators to frame

- Wire CAN buses

- Install Intel N95 PC, battery, IMU

- Run calibration scripts

### Phase 6: Software and Walking (Week 9-12)

- Install low-level control software

- Train RL policy in Isaac Lab (or use pretrained)

- Deploy to real robot

- TUNE, TUNE, TUNE

## QIDI Plus 4 Max Recommended Settings

### For Berkeley Humanoid Lite (PLA)

Nozzle: 0.4mm (stock)
Layer height: 0.2mm
Temperature: 210C / 60C bed
Chamber: 45C (heated chamber advantage!)
Walls: 4
Top/bottom: 5
Infill: 40% Gyroid
Speed: 50mm/s outer, 80mm/s inner
Support: Tree (auto)
Adhesion: Brim for large parts

### For High-Stress Parts (Optional PA-CF Upgrade)

Nozzle: 0.6mm hardened steel (for carbon fiber)
Temperature: 280-300C / 80C bed
Chamber: 60C (use heated chamber!)
Dry filament: PA-CF is hygroscopic — dry at 80C for 4+ hrs
Parts to print in PA-CF: Cycloidal gears, output shafts
Parts to keep in PLA: Housing (easier to print, less warping)

### QIDI Advantages for This Project

| Feature | Why It Matters |
| --- | --- |
| Heated chamber | Better layer adhesion for large structural parts |
| 350C nozzle | Enables PA-CF, PC for high-strength gears |
| Direct drive | Better extrusion control for abrasive filaments |
| Large build volume | Can print multiple parts at once |
| Multi-material | Could use different materials for different parts |

## Resources & Links

### Essential Links for Nick

| Resource | URL |
| --- | --- |
| Berkeley Humanoid Lite Main Repo | https://github.com/hybridrobotics/berkeley-humanoid-lite |
| BHL Documentation | https://berkeley-humanoid-lite.gitbook.io/docs/ |
| BHL BOM Spreadsheet | https://docs.google.com/spreadsheets/d/1AQEHcH_nPkXYfor2-h7bwNIUMmsePtAm53epnsWgZXc/edit |
| BHL Assets (CAD/URDF) | https://github.com/HybridRobotics/Berkeley-Humanoid-Lite-Assets |
| BHL Low-level Control | https://github.com/HybridRobotics/Berkeley-Humanoid-Lite-Lowlevel |
| BHL Releases | https://github.com/HybridRobotics/Berkeley-Humanoid-Lite/releases |
| BHL arXiv Paper | https://arxiv.org/abs/2504.17249 |
| InMoov | https://inmoov.fr/ |
| Poppy Humanoid | https://github.com/poppy-project/poppy-humanoid |
| ToddlerBot | https://github.com/hshi74/toddlerbot |
| ROBOTO ORIGIN | https://github.com/Roboparty/roboto_origin |
| AgiBot X1 | https://github.com/AgibotTech/agibot_x1_train |
| pib Robot | https://pib.rocks/ |
| K-Bot | https://github.com/kscalelabs/kbot |
| LeRobot | https://github.com/huggingface/lerobot |
| SO-100 Assembly | https://github.com/TheRobotStudio/SO-ARM100 |
| Reachy Mini | https://github.com/pollen-robotics/reachy_mini |
| Pollen Robotics | https://www.pollen-robotics.com/ |
| EngineAI GitHub | https://github.com/engineai-robotics/engineai_humanoid |
| Unitree G1 | https://www.unitree.com/products/g1 |

### Where to Buy Parts

| Vendor | Best For |
| --- | --- |
| AliExpress | Motors, bearings, hardware, electronics (China prices) |
| Amazon (US) | Faster shipping, verified BOM links |
| Mouser/DigiKey | B-G431B-ESC1 motor drivers, electronic components |
| RobotShop/Robotis | Dynamixel servos (for Poppy/ToddlerBot) |
| HobbyKing | LiPo batteries |
| MakerWorld | Community 3D print profiles |
| Onshape | Berkeley Humanoid Lite CAD (free to view/copy) |

### Communities

| Community | Platform | Link/Access |
| --- | --- | --- |
| Berkeley Humanoid Lite | Discord | Linked from GitHub |
| Berkeley Humanoid Lite | WeChat | China community |
| InMoov | MyRobotLab forums | https://myrobotlab.org/ |
| ToddlerBot | Discord | Linked from GitHub |
| ToddlerBot | WeChat | China community |
| pib | Discord | https://pib.rocks/ |
| K-Scale | Discord | https://docs.kscale.dev/ |
| LeRobot | Discord | Hugging Face server |
| ROBOTO ORIGIN | QQ Group | 546376843 |
| General humanoid | Reddit r/robotics | https://reddit.com/r/robotics |

Document compiled from extensive research of GitHub, arXiv, project websites, manufacturer documentation, and community resources. All information current as of July 2026.