# Physical Robots for AI Agent Integration: Comprehensive Research Report

Date: June 2026 Purpose: Identify physical robots that can integrate with AI agent systems, with specific models, prices, SDKs, open source code, protocols, and real-world deployment evidence.

## Table of Contents

- Unitree Robots (Quadruped + Humanoid)

- Tesla Optimus

- Food Delivery Robots

- Humanoid Robots in Deployment

- Layer 0 Protocols

- Open Source Robotics Code

- Real World Experiment Ideas

- Quick Reference Matrix

## 1. UNITREE ROBOTS

### Unitree Go2 ($1,600 - Most Accessible Quadruped)

| Spec | Value |
| --- | --- |
| Price | $1,600 (base) / ~$2,500 (EDU with Jetson Orin Nano) |
| Weight | 15 kg |
| Max Speed | 3.5 m/s |
| Battery | 8,000 mAh (approx 1-2 hours runtime) |
| DOF | 12 (3 per leg x 4) |
| Sensors | 4D LiDAR L1 (optional), Intel RealSense D435i depth camera, ultrasonic sensors, IMU |
| Compute | 8-core CPU (base) / NVIDIA Jetson Orin Nano 8GB (EDU) |
| Connectivity | WiFi 6, Bluetooth 5.2, Ethernet |
| IP Rating | IP54 |
| SDK | unitree_sdk2 (C++ + Python wrapper) |
| ROS2 Support | Full - unitree_ros2 package |
| Middleware | CycloneDDS |
| Availability | Buy today - ships globally |

SDK & Python Control:

# Official Python SDK
git clone https://github.com/unitreerobotics/unitree_sdk2_python.git
cd unitree_sdk2_python
pip3 install -e .

# Quick Start
from unitree_sdk2 import RobotInterface
robot = RobotInterface()
robot.start()
robot.move_forward(speed=0.2, duration=3.0)
robot.stand_up()
robot.sit_down()

# Read joint states
angles = robot.get_joint_angles()
vels = robot.get_joint_velocities()

# Read IMU
imu = robot.get_imu_data()

Key GitHub Repos: - unitreerobotics/unitree_sdk2 - Official C++ SDK (CycloneDDS-based) - unitreerobotics/unitree_sdk2_python - Python wrapper - unitreerobotics/unitree_ros2 - ROS2 support (Go2, B2, H1) - legion1581/go2_python_sdk - Unofficial Python SDK (DDS + WebRTC) - legion1581/go2_webrtc_connect - WebRTC connection library

Real World Use Cases Deployed NOW: - Conference guide/entertainer: WSO2 used Go2 as an AI agent at conferences - integrated with LLM for conversational guidance (source: wso2.com blog, Sept 2025) - University education: Teaching ROS2, SLAM, and robotics at 1000+ universities globally - Security patrol: Autonomous patrol in warehouses, campuses, parks - Research platform: RL locomotion research (10,000+ units in field)

### Unitree B2 (Industrial Quadruped)

| Spec | Value |
| --- | --- |
| Price | Contact for pricing (est. $25,000-$40,000) |
| Weight | 60 kg |
| Max Speed | 6 m/s (world’s fastest quadruped) |
| Payload | 120 kg (2:1 payload-to-weight ratio) |
| Battery | 5 hours unloaded / 4 hours with 20kg load |
| IP Rating | IP67 (waterproof, dustproof) |
| Operating Temp | -20C to +55C |
| Sensors | LiDAR, stereo cameras, ultrasonic, IMU |
| SDK | unitree_sdk2 (shared with Go2) |
| ROS2 | Full support |

Applications: Industrial inspection, power station patrol, emergency rescue, material transport, construction site logistics. Used in factories, warehouses, and outdoor industrial facilities worldwide.

### Unitree H1 Humanoid ($90,000)

| Spec | Value |
| --- | --- |
| Price | ~$90,000 (base) / ~$150,000 (H1-2) |
| Height | 180 cm (full human size) |
| Weight | 47 kg (H1) / 70 kg (H1-2) |
| Max Speed | 3.3 m/s RUNNING (world record for humanoid) |
| Knee Torque | 360 N.m (industry-leading) |
| Battery | 864 Wh, hot-swappable (1.5-2 hours) |
| DOF | 21 (H1) / 28 (H1-2 with 7-DOF arms) |
| Compute | Intel i5/i7 + optional Jetson Orin NX |
| Sensors | 3D LiDAR, depth camera, 360 perception |
| SDK | unitree_sdk2 + ROS2 |
| Simulation | NVIDIA Isaac Sim, MuJoCo |

### Unitree G1 Humanoid ($16,000 - Best Value Humanoid)

| Spec | Value |
| --- | --- |
| Price | $16,000 (base) - the most affordable humanoid |
| Height | 132 cm |
| Weight | 35 kg |
| DOF | 23-43 (configurable) |
| Dexterous Hands | Optional 7-9 DOF (EDU model) |
| Battery | 9000 mAh (~2 hours) |
| Compute | 8-core CPU + optional Jetson Orin |
| SDK | Full ROS2, Python, C++ |
| Simulation | Isaac Sim, MuJoCo |

Key Repos for Unitree + AI Integration:

https://github.com/unitreerobotics/unitree_sdk2
https://github.com/unitreerobotics/unitree_sdk2_python
https://github.com/unitreerobotics/unitree_ros2
https://github.com/unitreerobotics (main org - 30+ repos)

## 2. TESLA OPTIMUS

### Current Status (Gen 2 / Gen 3, 2026)

| Spec | Gen 2 (Current) | Gen 3 (Target) |
| --- | --- | --- |
| Price | N/A (internal only) | $20,000-$30,000 (target) |
| Height | 173 cm (5’8”) | Same body |
| Weight | 57 kg | Same |
| Walk Speed | 1.2 m/s | Same |
| Hand DOF | 11 DOF | 22 DOF (50 actuators) |
| Total Joints | 28 | 37 |
| Status | Internal factory use | Mass production started Jan 2026 |
| Availability | Not for sale | External sales: late 2026/2027 |

### Timeline

- Aug 2021: Tesla Bot announced at AI Day

- Dec 2023: Optimus Gen 2 unveiled (30% faster, 11-DOF hands)

- May 2024: Deployed at Tesla Fremont factory (battery sorting)

- Oct 2024: “We, Robot” event (teleoperated for crowd)

- Jan 2026: Gen 3 mass production begins at Fremont (Model S/X line converted)

- Q2 2026: Gen 3 hands deployed 24/7 in factory

- Late 2026: First external commercial deployments

- 2027 target: Consumer availability (Musk estimate)

### Tesla AI Stack

| Component | Technology |
| --- | --- |
| Perception | Tesla FSD (Full Self-Driving) vision stack - 8 cameras |
| Training | Dojo supercomputer (custom ASIC) |
| Inference | Tesla FSD Chip (HW4/HW5) |
| LLM/Voice | xAI Grok (voice AI integration confirmed) |
| Simulation | Tesla internal tools + real-world data |
| Control | End-to-end neural networks (learned from human demonstration) |

### Factory Deployment Evidence

- Sorting 4680 battery cells at Tesla Gigafactory

- Parts handling at Fremont (real tasks, not just demos)

- Manufacturing cost: $50,000-$100,000/unit currently

- Target: 1M robots/year at Fremont factory

- Giga Texas: Dedicated Optimus facility targeting 10M units/year

### Critical Notes

- Not commercially available as of mid-2026

- Multiple demos used teleoperation (not full autonomy)

- No independent peer-reviewed assessments published

- Competitors (Digit, Unitree G1) shipping TODAY

- Musk’s timelines consistently 1-3 years optimistic

## 3. FOOD DELIVERY ROBOTS

### Starship Technologies (World Leader)

| Metric | Value |
| --- | --- |
| Deliveries | 10 million+ autonomous deliveries (May 2026) |
| Fleet Size | 3,000+ robots across 8 countries |
| Universities | 60+ US campuses (1.5M students served) |
| Daily Road Crossings | 150,000+ per day |
| Miles Driven | 22+ million autonomous km |
| Autonomy Level | Level 4 (no human intervention in designated areas) |
| Founded | 2014 by Skype co-founders |
| Charge | $1.99-$2.99 per delivery |

Navigation Stack: - Proprietary autonomy stack (not open source) - Sensor fusion: cameras + GPS + IMU + wheel odometry - HD mapping of campus environments - Cloud-based fleet management - 99.8% delivery completion rate - Wireless charging (75% of campuses)

Key Partners: Uber Eats, Grubhub, Sodexo, Co-op, Tesco, foodora, Just Eat

Countries Active: US, UK, Germany, Switzerland, Sweden, Estonia, Finland

### Serve Robotics (NVIDIA-Powered)

| Metric | Value |
| --- | --- |
| Deliveries | 100,000+ autonomous deliveries |
| Cities | Los Angeles, Miami, Dallas, Atlanta, Chicago |
| Restaurants | 2,500+ |
| Completion Rate | 99.8% |
| Autonomy | Level 4 (designated areas) |
| Gen3 Speed | 11 mph (60% faster than Gen2) |
| Battery | 12+ hours per charge |
| Compute | NVIDIA Jetson Orin (5x compute boost) |
| Sensors | Ouster REV7 digital LiDAR |
| Manufacturing | Magna International (65% cheaper than Gen2) |
| Deployment | 2,000 robots by end of 2025 (Uber Eats contract) |

Serve Stack: - NVIDIA Isaac Sim for simulation - NVIDIA Jetson Orin for edge AI - 1M miles of data logged monthly (170B image-LiDAR samples) - Foundation model-driven navigation (post-Vayu Robotics acquisition)

### Nuro (Autonomous Delivery Vehicle)

| Metric | Value |
| --- | --- |
| Vehicle | R2, R3 (road-legal autonomous vehicles) |
| Payload | 80-180 kg |
| Speed | Up to 45 mph on roads |
| Status | Expanding to robotaxis and licensing |
| Backing | Google self-driving car veterans |
| Hardware | Nvidia + Arm for Nuro Driver |
| Powertrain | BYD electric motors/batteries |
| Regulation | Federal safety exemption (no mirrors/steering wheel) |

Key Shift (2024): Nuro branching into robotaxis and personally-owned autonomous vehicles - licensing Nuro Driver to car companies and rideshare operators.

### Coco Robotics

| Metric | Value |
| --- | --- |
| Founded | 2020, Santa Monica CA |
| Headcount | 500-1,000 |
| Partnership | DoorDash (Los Angeles, Chicago), RoboSense (LiDAR) |
| Pilots | Helsinki (100,000+ deliveries) |
| Payload | 10-20 kg |

### Top 10 Delivery Robot Brands (2025)

| Rank | Company | HQ | Key Product | Max Payload | Use Case |
| --- | --- | --- | --- | --- | --- |
| 1 | Starship | USA | Delivery Robot | 10 kg | Campus/city food |
| 2 | JD Logistics | China | Autonomous Van | 30-100 kg | E-commerce |
| 3 | Meituan | China | Delivery Robot | 10-30 kg | Food/grocery |
| 4 | Neolix | China | Autonomous Van | 100-200 kg | Smart city |
| 5 | Kiwibot | USA | Kiwibot 4.0 | 10 kg | Campus food |
| 6 | Nuro | USA | R2/R3 | 80-180 kg | Grocery/retail |
| 7 | HelloWorld | Malaysia | - | - | Campus logistics |
| 8 | Pudu | China | HolaBot/SwiftBot | 15-40 kg | Restaurant/office |
| 9 | Coco | USA | Scout-class | 10-20 kg | Sidewalk delivery |
| 10 | Fdata | China | Custom OEM | 30-150 kg | White-label B2B |

### Regulations by Region

USA: - No federal law specifically for delivery robots - State-level: California, Texas, Florida, Arizona allow sidewalk robots - Some cities (San Francisco) have banned sidewalk robots - Others actively recruit (Austin, Dallas, Miami) - NHTSA grants exemptions for vehicles without traditional controls

Europe: - UK: Milton Keynes world’s largest autonomous robot fleet - Germany: Hamburg operations; strict safety requirements - EU AI Act (2024): New requirements for AI-powered autonomous systems - GDPR compliance for camera data collection - Each country has different sidewalk/street permissions

China: - Most permissive: Chengdu, Beijing, Shanghai, Shenzhen grant road licenses - National strategy: mass-produce humanoids by 2025 - Smart city integration with 5G infrastructure - Government actively funding autonomous delivery

## 4. HUMANOID ROBOTS IN DEPLOYMENT

### Figure AI / Figure 02 at BMW (REAL DEPLOYMENT)

| Metric | Value |
| --- | --- |
| Location | BMW Group Plant Spartanburg, South Carolina |
| Duration | 11 months continuous |
| Shifts | 10-hour shifts, Monday-Friday |
| Parts Moved | 90,000+ sheet metal components |
| Vehicles Produced | 30,000+ BMW X3 |
| Runtime | 1,250+ hours |
| Steps Taken | 1.2+ million |
| Task | Sheet metal loading (pick-and-place) |
| Accuracy | 5mm tolerance placement |
| Cycle Time | 84 seconds total, 37 seconds load |
| KPI Success | Target: 99% accuracy, 0 interventions per shift |

Key Learning: Transition from lab to production was FASTER than expected. Motion sequences transferred quickly into stable shift operation. Integration via standardized interfaces into BMW Smart Robotics ecosystem.

Figure 03: Released Nov 2025. Redesigned wrist electronics (direct motor controller communication), improved forearm reliability (top failure point at BMW).

### Agility Digit at Amazon + GXO (First Commercial Humanoid)

| Spec | Value |
| --- | --- |
| Price | Target <$50,000 |
| Height | 175 cm |
| Weight | 65 kg |
| Payload | 16 kg per hand |
| Walk Speed | 5.1 km/h |
| Battery | ~2-3 hours |
| DOF | 28 |
| Manufacturing | RoboFab (Salem, OR) - 10,000/year capacity |

Deployments: - Amazon: Testing tote recycling at R&D center near Seattle (2023) - GXO/Spanx: First commercial humanoid deployment in production (2024) - Tote-moving tasks in warehouse - “First humanoid deployed at a customer site, generating revenue”

### Apptronik Apollo (NASA Heritage)

| Spec | Value |
| --- | --- |
| Height | 173 cm (5’8”) |
| Weight | 72.6 kg |
| Payload | 25 kg |
| Battery | 4 hours hot-swappable |
| DOF | 44 |
| Config | Stationary / wheeled / bipedal (modular) |
| Max Speed | 1.2-1.5 m/s walking |
| Actuators | Proprietary linear electric (13+ generations) |
| Heritage | NASA Valkyrie robot project |

Partnerships: - Jabil: Scaling production + manufacturing deployment (Feb 2025) - NASA: Valkyrie humanoid project heritage - Designed for logistics, manufacturing, human-safe collaboration

### Chinese Humanoid Robots

#### UBTech Walker X

- Price: Not publicly disclosed (est. $50,000+)

- DOF: 41 (record for service robots)

- Height: 145 cm

- Battery: 2 hours (major limitation)

- Speed: 3 km/h

- Deployment: Shopping centers, hotels, public spaces in China/Southeast Asia

- Features: Smart home integration (Xiaomi, Huawei, Google Home)

- Limitation: Proprietary SDK, closed ecosystem, limited European support

#### Fourier Intelligence GR-1

- Price: ~$125,000

- Height: 165 cm

- Weight: 55 kg

- DOF: 40

- Max Speed: 5 km/h walk, 3 km/h normal

- Payload: 50 kg (very strong for its class)

- Market: Healthcare, rehabilitation, research

- Status: Production version launched Oct 2024

- Country: China

#### Xiaomi CyberOne

- Price: $89,000-$104,000 (not commercially available)

- Height: 177 cm

- Weight: 52 kg

- DOF: 21

- Speed: 3.6 km/h walking, 7.2 km/h running

- Payload: 1.5 kg per hand (weak)

- Special: Emotion recognition (45 classes), 85 sound types

- Features: Curved OLED face, Mi-Sense depth vision

- Status: R&D platform only, no mass production timeline

#### Other Notable Chinese Humanoids

| Robot | Price | Key Feature |
| --- | --- | --- |
| AGIBOT A2 Ultra | $100,000-$190,000 | Available now |
| Unitree R1 | $4,900 | Pre-order (ships April 2026) |
| Noetix Bumi | $1,400 | Hobbyist/entry level |
| NEURA 4NE1 | EUR 19,999-98,000 | Porsche collaboration |
| EngineAI T800 | $25,000 | Available now |

### Price History: Humanoid Robot Cost Drop

$2.5M  Honda ASIMO (2000)
$1M+   Boston Dynamics Atlas (2013)
$25K    SoftBank Pepper (2015)
$90K    Unitree H1 (2024)
$16K    Unitree G1 (2024)
$4.9K   Unitree R1 (2025)

99.4% price drop in 25 years
Projected $5K by 2030

## 5. LAYER 0 PROTOCOLS

The foundational communication protocols that enable robots to operate, communicate, and integrate with AI agents.

### ROS2 (Robot Operating System 2) - The De Facto Standard

| Attribute | Value |
| --- | --- |
| Type | Open-source robotics middleware |
| License | Apache 2.0 |
| Current LTS | Jazzy Jalisco (May 2024 - May 2029) |
| Previous LTS | Humble Hawksbill (May 2022 - May 2027) |
| Latest | Lyrical Luth (May 2026 - May 2031) |
| Underlying | DDS (Data Distribution Service) |
| Languages | Python, C++ |
| Stars | The core is used by 10,000+ repos |

ROS2 Distributions (2026):

| Distro | Release | Type | EOL | Ubuntu | Python |
| --- | --- | --- | --- | --- | --- |
| Humble | May 2022 | LTS | May 2027 | 22.04 | 3.10 |
| Jazzy | May 2024 | LTS | May 2029 | 24.04 | 3.12 |
| Kilted | May 2025 | Standard | Dec 2026 | 24.04 | 3.12 |
| Lyrical | May 2026 | LTS | May 2031 | 26.04 | 3.13 |

Key ROS2 Components for AI Agents:

Nodes: Independent computation units (AI models, sensors, actuators)
Topics: Pub/sub message channels (sensor data, commands)
Services: Request/response RPC
Actions: Long-running tasks with feedback
Nav2: Navigation stack (SLAM, path planning, obstacle avoidance)
MoveIt 2: Motion planning for manipulators
ros2_control: Hardware abstraction layer

Python Example:

import rclpy
from rclpy.node import Node
from sensor_msgs.msg import Image
from geometry_msgs.msg import Twist

class AIAgentNode(Node):
    def __init__(self):
        super().__init__('ai_agent')
        self.publisher_ = self.create_publisher(Twist, '/cmd_vel', 10)
        self.subscription = self.create_subscription(
            Image, '/camera/image_raw', self.image_callback, 10)
        self.timer = self.create_timer(0.1, self.control_loop)
    
    def image_callback(self, msg):
        # AI agent processes image
        pass
    
    def control_loop(self):
        cmd = Twist()
        cmd.linear.x = 0.5  # Move forward
        self.publisher_.publish(cmd)

rclpy.init()
node = AIAgentNode()
rclpy.spin(node)

### DDS (Data Distribution Service) - Real-Time Pub/Sub

| Attribute | Value |
| --- | --- |
| Standard | OMG (Object Management Group) |
| Protocol | RTPS (Real-Time Publish-Subscribe) over UDP/IP |
| Implementations | CycloneDDS, FastDDS (eProsima), RTI Connext, OpenDDS |
| Key Feature | Decentralized discovery, no broker needed |
| QoS | Configurable: reliability, durability, deadline, lifespan |
| Use Case | Real-time robot-to-robot communication |

ROS2 DDS Implementations: - FastDDS (default in Humble/Jazzy) - CycloneDDS (used by Unitree, Eclipse foundation) - RTI Connext (commercial, high-performance) - Zenoh (emerging alternative, Tier 1 in Kilted+)

Unitree SDK2 is built directly on CycloneDDS - communicates without ROS2 running, but is ROS2-compatible via DDS interop.

### MQTT (Message Queuing Telemetry Transport) - IoT Messaging

| Attribute | Value |
| --- | --- |
| Type | Lightweight pub/sub messaging |
| Transport | TCP/IP |
| Overhead | Minimal (2-byte fixed header) |
| Pattern | Client-Broker (centralized) |
| QoS Levels | 0 (at most once), 1 (at least once), 2 (exactly once) |
| Best For | Low-power sensors, cloud connectivity, fleet management |

MQTT for Robots:

import paho.mqtt.client as mqtt

client = mqtt.Client()
client.connect("mqtt.broker.com", 1883)

# Publish sensor data
client.publish("robot/go2/battery", "87%")
client.publish("robot/go2/position", "{x: 1.2, y: 3.4}")

# Subscribe to commands
client.subscribe("robot/go2/cmd")

def on_message(client, userdata, msg):
    if msg.topic == "robot/go2/cmd":
        execute_command(msg.payload)

client.on_message = on_message
client.loop_start()

Use Cases: Cloud fleet monitoring, over-the-air updates, telemetry, cross-facility coordination.

### AMQP (Advanced Message Queuing Protocol) - Enterprise Messaging

| Attribute | Value |
| --- | --- |
| Type | Enterprise message queuing |
| Pattern | Exchange -> Queue -> Consumer |
| Features | Message persistence, transactions, complex routing |
| Implementations | RabbitMQ, Apache Qpid, Azure Service Bus |
| Best For | Backend systems, order processing, multi-robot task allocation |

MQTT vs AMQP for Robotics: - MQTT: Use on robot (lightweight, embedded-friendly) - AMQP: Use in backend (reliable, complex routing, persistence) - Together: Robot publishes via MQTT -> Gateway converts to AMQP -> Backend processes

### OPC-UA (Open Platform Communications Unified Architecture) - Industrial Standard

| Attribute | Value |
| --- | --- |
| Type | Industrial automation communication |
| Standard | IEC 62541 |
| Foundation | OPC Foundation (678+ members: Siemens, ABB, Rockwell, Microsoft) |
| Architecture | Client-Server + Pub/Sub |
| Security | Built-in encryption, certificates, authentication |
| Data Modeling | Semantic data models (companion specs for robots, CNC, etc.) |
| Best For | Factory automation, PLC integration, Industry 4.0 |

OPC-UA for Robots: - Companion specification for robotics - Direct PLC integration without custom gateways - Semantic data model (not just raw values - knows what the data means) - Publish/Subcribe over MQTT (PubSub over MQTT spec) - Time-Sensitive Networking (TSN) support for real-time

### CAN bus (Controller Area Network) - Vehicle/Internal Network

| Attribute | Value |
| --- | --- |
| Type | Multi-master serial bus |
| Speed | Up to 1 Mbps (CAN), 5-8 Mbps (CAN-FD) |
| Max Nodes | 128 (theoretical), typically < 32 |
| Distance | Up to 40m at 1 Mbps |
| Topology | Differential twisted pair |
| Best For | Motor controllers, sensors, embedded real-time control |

CAN bus in Robotics: - Internal robot bus for motor drivers, IMU, encoders - Used by Unitree Go2 (internal leg motor communication) - Standard in automotive (Tesla, Nuro) - Real-time with priority-based arbitration - Every motor controller, sensor publishes on the bus

Motor Controller 1 <--->
Motor Controller 2 <---> CAN bus <---> Main Computer
Motor Controller 3 <--->
IMU Sensor     <--->

### EtherCAT - Real-Time Industrial Ethernet

| Attribute | Value |
| --- | --- |
| Type | Real-time industrial Ethernet |
| Speed | 100 Mbps |
| Cycle Time | < 100 microseconds |
| Topology | Line/pass-through (each node forwards frame) |
| Protocol | Master-Slave |
| Best For | High-speed synchronized motion control, CNC, factory automation |

EtherCAT for Robots: - Industrial robot arm control (KUKA, FANUC, Universal Robots) - Synchronized multi-axis motion - Used in Agility Digit, Figure 03 manufacturing - Not typical for mobile robots (CAN bus preferred there)

### RTPS (Real-Time Publish-Subscribe) - DDS Wire Protocol

| Attribute | Value |
| --- | --- |
| Type | Wire protocol for DDS |
| Standard | OMG DDSI-RTPS |
| Transport | UDP/IP (multicast for discovery) |
| Key Feature | Automatic discovery - no central broker |
| QoS | Configurable for each topic |
| Use Case | Robot-to-robot real-time data exchange |

RTPS enables: - Automatic node discovery (robots find each other on the network) - Decentralized communication (no single point of failure) - Configurable QoS per topic (some need reliability, some need speed)

### gRPC - Robot-to-Cloud Communication

| Attribute | Value |
| --- | --- |
| Type | High-performance RPC framework |
| Transport | HTTP/2 |
| Serialization | Protocol Buffers (binary, efficient) |
| Patterns | Unary, server streaming, client streaming, bidirectional |
| Best For | Robot-to-cloud API calls, multi-robot fleet management |

gRPC for Robots:

// Robot service definition
service RobotService {
  rpc GetStatus(RobotId) returns (RobotStatus);
  rpc StreamSensorData(RobotId) returns (stream SensorData);
  rpc SendCommand(Command) returns (CommandResult);
  rpc BidirectionalControl(stream Command) returns (stream State);
}

### Eclipse Zenoh - The Emerging Alternative

| Attribute | Value |
| --- | --- |
| Type | Next-gen pub/sub/query protocol |
| Features | Pub/Sub + Queries + Storage |
| Transport | TCP, UDP, QUIC, WebSockets, Bluetooth, serial |
| Best For | Cloud-to-edge, multi-robot fleets, IoT integration |
| Status | Tier 1 middleware in ROS2 Kilted+ |

Zenoh advantages: - Works over any transport (not just Ethernet) - Internet-native (not just LAN like DDS) - Less configuration than DDS - No XML config files

### How AI Agents Interface with These Protocols

AI AGENT LAYER:
  LLM (Grok, GPT-4, Claude) --> Natural language commands
  VLA Models (Pi0, GR00T) --> Vision-to-action
  Planning (PDDL, LLM-based) --> Task decomposition

ROBOT MIDDLEWARE LAYER:
  ROS2 / DDS / Zenoh --> Node communication
  Nav2 --> Navigation, SLAM, path planning
  MoveIt 2 --> Arm motion planning
  ros2_control --> Hardware abstraction

HARDWARE PROTOCOL LAYER:
  CAN bus --> Motor control, encoders, IMU
  EtherCAT --> High-speed synchronized motion
  WiFi/Ethernet --> Video streaming, LiDAR data
  Bluetooth --> Low-bandwidth sensors

CLOUD/INTEGRATION LAYER:
  gRPC --> Robot-to-cloud API
  MQTT --> Fleet telemetry, OTA updates
  OPC-UA --> Factory PLC integration
  AMQP --> Backend task allocation

## 6. OPEN SOURCE ROBOTICS CODE

### Simulation

| Repository | Stars | License | Last Update | Description |
| --- | --- | --- | --- | --- |
| isaac-sim/IsaacSim | 3.5K | Apache 2.0 | June 2026 | NVIDIA Isaac Sim - GPU-accelerated robot simulation |
| gazebosim/gz-sim | 410 | Apache 2.0 | June 2026 | Gazebo - de facto standard robot simulator |
| Unity-Technologies/Unity-Robotics-Hub | 2.5K | Apache 2.0 | Nov 2024 | Unity Robotics tools |
| google-deepmind/mujoco | 9.5K+ | Apache 2.0 | Active | MuJoCo physics engine (acquired by Google) |

### SLAM & Navigation

| Repository | Stars | License | Last Update | Description |
| --- | --- | --- | --- | --- |
| UZ-SLAMLab/ORB_SLAM3 | 6.5K | GPLv3 | Active | Visual, visual-inertial, multi-map SLAM |
| TixiaoShan/LIO-SAM | 4.5K | BSD | Active | Lidar-inertial odometry via smoothing/mapping |
| HKUST-Aerial-Robotics/FAST_LIO | 3.2K | BSD | Active | Fast LiDAR-inertial odometry |
| PRBonn/kinematic-icp | 200+ | MIT | Active | Kinematic ICP for SLAM |
| ros-planning/navigation2 | 2.3K | Apache 2.0 | Active | Nav2 for ROS2 - navigation stack |

### Robot Learning

| Repository | Stars | License | Last Update | Description |
| --- | --- | --- | --- | --- |
| huggingface/lerobot | 24.1K | Apache 2.0 | Active | End-to-end robot learning (imitation + RL + VLA) |
| nvidia/IsaacLab | 3.2K | BSD-3 | Active | NVIDIA Isaac Lab - RL for robotics |
| roboterax/humanoid-gym | 800+ | MIT | Active | RL for humanoid zero-shot sim2real |
| jonyzhang2023/awesome-humanoid-learning | 1.5K | - | Active | Curated humanoid learning resources |

### Computer Vision for Robots

| Repository | Stars | License | Last Update | Description |
| --- | --- | --- | --- | --- |
| ultralytics/ultralytics | 35K+ | AGPL 3.0 | Active | YOLOv8 - object detection for robots |
| monkeyrom/3D_Object_Detection_and_Pose_Estimation_for_Automated_Bin-Picking_Application | 100+ | - | Active | YOLOv5 + FAST/BRISK for bin picking |
| Walid-khaled/YOLO-Object-Detection-for-Pick-and-Place-task-using-ROS-on-KUKA-iiwa | 200+ | - | Active | YOLOv4 pick-and-place with ROS |

### Unitree / Quadruped Control

| Repository | Stars | License | Last Update | Description |
| --- | --- | --- | --- | --- |
| unitreerobotics/unitree_sdk2 | 500+ | - | Active | Official C++ SDK for Go2/B2/H1 |
| unitreerobotics/unitree_sdk2_python | 400+ | - | Active | Python SDK wrapper |
| unitreerobotics/unitree_ros2 | 300+ | - | Active | ROS2 support |
| legion1581/go2_python_sdk | 200+ | MIT | Active | Unofficial Python SDK (DDS) |
| legion1581/go2_webrtc_connect | 150+ | MIT | Active | WebRTC connection for Go2 |

### Humanoid Control

| Repository | Stars | License | Last Update | Description |
| --- | --- | --- | --- | --- |
| roboterax/humanoid-gym | 800+ | MIT | Active | RL training for humanoid sim2real |
| RoboMasters/China-Open-2024 | Various | - | 2024 | RoboMaster humanoid competition |

### Key Open-Source Humanoid Projects

| Project | Type | Hardware | Cost |
| --- | --- | --- | --- |
| SO-ARM100 (LeRobot) | Robot arm | 3D-printed + servo motors | ~$200 |
| Koch v1.1 | Robot arm | 3D-printed | ~$250 |
| LeKiwi | Mobile manipulator | 3D-printed + wheels | ~$350 |
| OpenLoong | Humanoid | 3D-printed biped | ~$1,000 |
| Axon | Humanoid | Raspberry Pi + ESP32 | ~$500 |
| SMART Humanoid | Full humanoid | Modular, research-grade | $5,000+ |

## 7. REAL WORLD EXPERIMENT IDEAS

### CSOAI Governance Experiments with Physical Robots

#### Experiment 1: “When a Delivery Robot Crosses from EU to UK Post-Brexit”

Setup: - Deploy Starship robot on a route that crosses Northern Ireland / Republic of Ireland border - Or simulate: same robot model operating in both jurisdictions

Governance Questions: - Which safety regulations apply? EU Machinery Directive vs UK CA marking? - GDPR applies in EU - what camera data can be collected in ROI vs NI? - Liability: if robot causes accident at the border, which legal system applies? - Insurance requirements differ - how does the robot “know” which rules apply?

Technical Implementation:

# Robot carries a "jurisdiction registry"
class RobotGovernance:
    jurisdictions = {
        "EU": {"gdpr": True, "safety_std": "EN_ISO_12100", "speed_limit": 6},
        "UK": {"gdpr": False, "safety_std": "BS_EN_ISO_12100", "speed_limit": 4},
    }
    
    def on_gps_position_update(self, lat, lon):
        current_jurisdiction = self.get_jurisdiction(lat, lon)
        self.apply_rules(current_jurisdiction)
        self.log_compliance(current_jurisdiction)

#### Experiment 2: “Multi-Jurisdiction Humanoid Workplace Safety”

Setup: - Same humanoid robot (Unitree H1 or similar) operating in: - USA (OSHA regulations) - Germany (DGUV/ISO standards) - China (GB standards)

Governance Questions: - Safety zone distances differ by country - how to reconfigure? - Emergency stop requirements vary - Worker notification/consent requirements differ - Documentation and audit trails have different requirements

Technical Implementation: - Robot loads jurisdiction-specific safety module on boot - AI agent monitors compliance and generates audit logs - Dynamic reconfiguration based on GPS + facility database

#### Experiment 3: “Autonomous Vehicle Liability Attribution”

Setup: - Delivery robot fleet with mixed autonomy levels - Log every decision point where AI agent vs. human operator has control

Governance Questions: - When robot is in AI mode: who is liable for accident? - When human teleoperator intervenes: does liability shift? - How to prove (in court) who was in control at moment of incident? - What logging/evidence standards are required?

Technical Implementation: - Blockchain-based decision logging (immutable) - Real-time control mode recording - Automatic incident report generation

#### Experiment 4: “Cross-Border Robot Data Governance”

Setup: - Robot collects camera/LiDAR data in multiple countries - Data processed in cloud (potentially different country) - Training data used to improve models (potentially different country again)

Governance Questions: - Camera data from German streets - can it be used to train models in US? - GDPR “right to be forgotten” - how to remove a person from training data? - Data residency requirements by country - Cross-border AI model transfer restrictions

#### Experiment 5: “Robot Governance Compliance Automation”

Setup: - AI agent continuously monitors robot operations against regulatory database - Auto-generates compliance reports - Flags potential violations before they occur

Architecture:

Robot Sensors --> AI Agent --> Regulatory Knowledge Graph
                                  |
                                  v
                           Compliance Monitor
                                  |
                    +-------------+-------------+
                    |             |             |
                    v             v             v
               Auto-report   Alert human   Reconfigure
               generation    operator      robot

## 8. QUICK REFERENCE MATRIX

### Robot Selection for AI Agent Integration

| Use Case | Robot | Price | SDK Quality | ROS2 | Availability |
| --- | --- | --- | --- | --- | --- |
| Education/Research | Unitree Go2 EDU | $2,500 | Excellent | Full | Buy now |
| Low-cost humanoid | Unitree G1 | $16,000 | Excellent | Full | Buy now |
| Industrial inspection | Unitree B2 | $30K est. | Good | Full | Buy now |
| Delivery (build on) | Reference: Starship | N/A | N/A | N/A | Partner only |
| Warehouse automation | Agility Digit | <$50K | Good | Yes | Pilot program |
| Factory integration | Figure 03 | TBA | Good | Yes | Late 2026 |
| Wait-and-see | Tesla Optimus | $20-30K (target) | Unknown | Unknown | 2027+ |
| Healthcare/service | Fourier GR-1 | $125K | Limited | Partial | Early access |

### Protocol Selection Matrix

| Need | Protocol | When to Use |
| --- | --- | --- |
| Robot application development | ROS2 | Always (de facto standard) |
| Real-time robot-to-robot comms | DDS/RTPS | Multi-robot systems |
| Motor control, embedded | CAN bus | Internal robot bus |
| Industrial PLC integration | OPC-UA | Factory automation |
| Cloud connectivity, telemetry | MQTT | Fleet management, IoT |
| Backend task allocation | AMQP | Enterprise multi-robot |
| High-speed motion control | EtherCAT | Industrial robot arms |
| Robot-to-cloud API | gRPC | Fleet management systems |
| Next-gen multi-robot | Zenoh | Cloud-to-edge, 2025+ |

## SOURCES & REFERENCES

### Key URLs

Unitree: - https://github.com/unitreerobotics - https://support.unitree.com/home/en/developer - https://www.unitree.com

Tesla Optimus: - https://www.tesla.com/optimus - https://www.figure.ai - https://blog.robozaps.com/b/tesla-optimus-gen-2-review

Delivery Robots: - https://www.starship.xyz - https://serverobotics.com - https://www.nuro.ai - https://www.cocodelivery.com

Open Source: - https://github.com/huggingface/lerobot (24.1K stars) - https://github.com/isaac-sim/IsaacSim (3.5K stars) - https://github.com/UZ-SLAMLab/ORB_SLAM3 (6.5K stars) - https://github.com/TixiaoShan/LIO-SAM (4.5K stars)

Protocols: - https://docs.ros.org/en/jazzy/ - https://opcfoundation.org/about/what-is-opc/ - https://mqtt.org - https://zenoh.io

Report compiled: June 2026 For CSOAI governance experiment planning